PHARMACY DATA MANAGEMENT

TECHNICAL MANUAL/
SECURITY GUIDE

Version 1.0

September 1997

(Revised February 2011)

Department of Veterans Affairs

Product Development

Revision History

Each time this manual is updated, the Title Page lists the new revised date and this page describes the changes. If the Revised Pages column lists “All,” replace the existing manual with the reissued manual. If the Revised Pages column lists individual entries (e.g., 25, 32), either update the existing manual with the Change Pages Document or print the entire new manual.

| Date | Revised Pages | Patch Number | Description |
| --- | --- | --- | --- |
| 02/11 | i, 25 | PSS*1*151 | Added PSSDSAPA routine to the Routine List. Released with CPRS version 28. (G. Tucker, PM; C. Bernier, Tech Writer) |
| 02/11 | i, 33-34 | PSS*1*142 | Added functionality to denote the default med route for IV orders in the selection list in CPRS if all of the orderable items on the order have the same default med route defined. Released with CPRS version 28. (N. Goyal, PM; E. Phelps, Tech Writer) |
| 02/10 | i-iii, 5, 8, 24f, 25, 29-30, 31b- 32, 32a-32h, 45-50 | PSS*1*147 | Updated patch references to include PSS*1*147. Described files, fields, options and routines added/modified as part of this patch. (J. Freese, DM; K. Lowry, Tech. Writer) |
| 10/09 | i-ii, 5, 8 | PSS*1*141 | Updated patch references to include PSS*1*141. Added ASSOCIATED IMMUNIZATION field (#9) to the PHARMACY ORDERABLE ITEM file (#50.7). (R. Singer, DM; D. Dertien/R. Silverman, Tech Writer) |
| 08/09 | 24f | PSS*1*140 | Added new option Default Med Route OI Rpt [PSS DEF MED ROUTE OI RPT]. (E. Wright, PM; S. B. Scudder, Tech. Writer) |
| 02/09 | i-ii, 24b-f, 25, 29-31b, 48-52, 55-58 | PSS*1*129 | Pharmacy Re-Engineering (PRE) V.0.5 Pre-Release. Restructured main PSS MGR menu and added new Enhanced Order Checks Setup Menu. Described files, fields, options and routines added/modified as part of this project. (G.Tucker, PM; V.Howell, Tech.Writer) |
| 08/08 | iii, 25, 33-34 | PSS*1*94 | Added Medication Routes and Administration Scheduling sections. Added PSSSCHED routine. (S. Templeton, PM; G. O’Connor, Tech. Writer) |
| 10/06 | i, ii, 25 | PSS*1*112 | Pharmacy Re-Engineering (PRE) Encapsulation Cycle II project. Added routines PSS55MIS and PSS50TMP to the Routine List. (J. Caudullo, PM; J. Nemetz, Tech. Writer) |
| 09/06 | i, ii, 25 | PSS*1*108 | Pharmacy Re-Engineering (PRE) Encapsulation Cycle II project. Added routine PSS551 to the Routine List. (H. Whitley, PM; J. Nemetz, Tech. Writer) |
| 04/06 | i, 25 | PSS*1*90 | HIPAA NCPDP Global project. Added routines PSSDAWUT and PSSNDCUT to the Routine List. (S. Spence, PM; M. Newman, Tech. Writer) |
| 04/06 | i, ii, 25 | PSS*1*106 | Pharmacy Re-Engineering (PRE) Encapsulation Cycle II project. Added routine PSS781 to the Routine List. (H. Whitley, PM; L. Woodson, Tech. Writer) |
| 11/05 | i, ii, 25 | PSS*1*101 | Pharmacy Re-Engineering (PRE) Encapsulation Cycle II project. Added routines PSS55 and PSS59P7 to the Routine List. (H. Whitley, PM; L. Woodson, Tech. Writer) |
| 03/05 | i, ii, 24a, 25, 29-31, 48 | PSS*1*87 | Laser Labels Phase II project. Added Warning Builder and Warning Mapping options descriptions and updated the menu options. Added four new routines to the routine list. Cleaned up misspelled words and such on many pages. (H. Whitley, PM; L. Woodson, Tech. Writer) |
| 10/04 | i., 25, 33 | PSS*1*85 | Added routines and a reference to the Pharmacy Re-Engineering (PRE) Application Program Interface (API) Manual created for the Pharmacy Re-Engineering (PRE) project Encapsulation cycle 1. |
| 10/04 | i, 24a, 25, 29-31, 32d-h, 48, 53 | PSS*1*82 | Updated the option description to include Send Entire Drug File to External Interface [PSS MASTER FILE ALL] option. Added new master file update information to the “HL7 Messaging with an External System” section. Updated routine list to include PSSMSTR. Updated the web address for the VistA Documentation Library (VDL). |
| 07/03 | i, 25, 31, 48 | PSS*1*61 | Updated routine list to four new add PKI routines. Added new Controlled Substances/PKI Reports [PSS/PKI REPORTS] menu and four associated report options to the Pharmacy Data Management [PSS MGR] menu. |
| 04/03 | i, 5, 8, 29, 35, 48 | PSS*1*68 | Updated patch references to include PSS*1*68. Added NON-VA MED field (#8) to the PHARMACY ORDERABLE ITEM file (#50.7). |
| 03/03 | i., 5, 8, 24a, 29, 31, 35, 48 | PSS*1*47 | Updated patch references to include PSS*1*47. Added new field OTHER LANGUAGE INSTRUCTIONS (#7.1) to the PHARMACY ORDERABLE ITEM file (#50.7) list and Other Language Translation Setup option description. |
| 11/02 | i, ii 5, (6) 23 - 25, (26) 29-30,(47), 48 | PSS*1*55 | Renumbered front matter starting from this Revision History page. Updated Patch number. Updated Option descriptions to include Drug Text File Report option. Added routine PSSDTR in the Routines section. Added the Drug Text File Report option to the current PDM Menu in the Exported Options section. |
| 10/02 | Title, i-iv, 32a-32d | PSS*1*57 | Updated Title Page, Revision Page and Table of Contents. A section was added for the new HL7 Messaging with an External System. |
| 09/01 | All | PSS*1*38 | Added this Revision History Page. Added Patch Release changes and Pharmacy Ordering Enhancements (POE) edits. Updated manual to comply with current documentation standards. |
| 09/97 | N/A | N/A | Original Release of Technical Manual. |

(This page included for two-sided copying.)

Table of Contents

Introduction	1

Technical Manual	3

Implementation and Maintenance	5

Work Flowchart for Creating and Matching Orderable Items	9

Steps in Creating the Pharmacy Orderable Item File	11

File List	13

Option Descriptions	17

Routines	25

Exported Options	27

Protocols	35

HL7 Messaging with an External System	35

Data Archiving and Purging	33

Callable Routines/Entry Points/Application Program Interfaces (APIs)	33

Medication Routes	33

Administration Scheduling	33

External Relations	34

Internal Relations	34

Package-Wide Variables	34

Package Requirements	35

Additional Information	37

Security Guide	43

Security Management	45

Mail Groups	45

Alerts	45

Remote Systems	45

Archiving/Purging	45

Contingency Planning	45

Interfacing	46

Electronic Signatures	46

Menus	47

Security Keys	50

File Security	51

References	53

Glossary	55

(This page included for two-sided copying.)

# Introduction

Pharmacy Data Management (PDM) provides tools for managing Pharmacy data. It includes tools for creating Pharmacy Orderable Items and maintaining files necessary for the Computer Patient Record System (CPRS). PDM consolidates tools for managing the various Pharmacy software products. It provides Pharmacy Supervisors, in one location, the capability to enter and edit data from the local DRUG file (#50) for all Pharmacy related packages.

The PDM Technical Manual is designed to acquaint the user with the various PDM options and offer specific guidance on the maintenance and use of the PDM package. Documentation concerning the PDM package, including any subsequent change pages affecting this documentation, can be found at the VistA Documentation Library (VDL) on the Veterans Administration Intranet at http://vista.med.va.gov/vdl.

Notations that will be used consistently throughout this PDM Technical Manual are outlined below.

- Menu options will be italicized.

Example: The Drug Enter/Edit option permits you to enter or edit a drug.

- Screen prompts will be denoted with quotation marks around them.

Example: the "SELECT DRUG" prompt will display next.

- Responses in bold face indicate user input.

Example: DRUG INTERACTION SEVERITY: CRITICAL

- Text centered between bent parentheses represents a keyboard key that needs to be pressed in order for the system to capture a user response or move the cursor to another field.

<Enter> indicates that the Enter key (or Return key on some keyboards) must be pressed.

Example: Type Y for Yes or N for No and press <Enter>

<Tab> indicates that the Tab key must be pressed.

Example:  Press <Tab> to move the cursor to the next field.

- Indicates especially important or helpful information.

- Options are locked with a particular security key.  The user must hold the particular security key to be able to perform the menu option.

Example:     Without the PSXCOMPMGR key the Consolidated Mail Outpatient Pharmacy options cannot be accessed.

-  The page symbol indicates a referral to a diagram.

- ?, ??, ???  One, two or three question marks can be entered at any of the prompts for online help. One question mark elicits a brief statement of what information is appropriate for the prompt. Two question marks provide more help, plus the hidden actions, and three question marks will provide more detailed help, including a list of possible answers, if appropriate.

- ^  Up arrow (caret or a circumflex) and pressing <Enter> can be used to exit the present option.

Technical Manual

(This page included for two-sided copying.)

## Implementation and Maintenance

The PHARMACY ORDERABLE ITEM file (#50.7) must be built prior to the installation of Outpatient Pharmacy V. 7.0 and Inpatient Medications V. 5.0. The PHARMACY ORDERABLE ITEM file (#50.7) will be similar to the PRIMARY DRUG file (#50.3) used by Inpatient Medications V. 4.5. The main difference is that each entry in the PHARMACY ORDERABLE ITEM file (#50.7) has an associated Dosage Form that will always print next to the name. The PHARMACY ORDERABLE ITEM file (#50.7) will be duplicated in the ORDERABLE ITEM file (#101.43) that will reside in CPRS V. 1.0. Any update to the PHARMACY ORDERABLE ITEM file (#50.7) will automatically update the corresponding entry in the ORDERABLE ITEM file (#101.43). The NAME field (#.01) and the DOSAGE FORM field (#.02) values in the PHARMACY ORDERABLE ITEM file (#50.7) will print on Outpatient Pharmacy reports, profiles, Inpatient Medication reports, MAR labels etc., in place of the Primary Drug Name.

The PHARMACY ORDERABLE ITEM file (#50.7) includes the following fields. These fields reflect the package content up to and including the release of patch PSS*1*147.

 These fields were necessary for the initial installation but were deleted following later patches.

* These fields were not exported with the initial installation but were added with later patches.

| Field Number |  | #### Field Name | Description |
| --- | --- | --- | --- |
| .01 |  | #### NAME | This is the name of the Pharmacy Orderable Item. It is a free text field that can be up to 40 characters in length. |
| .02 |  | DOSAGE FORM | This is a pointer to the DOSAGE FORM file (#50.606). This is a required field and will always print next to the Pharmacy Orderable Item name. |
| .03 |  | IV FLAG | This field will be set to 1 to indicate that the Pharmacy Orderable Item entry is pointed to by either the IV ADDITIVES file (#52.6), or the IV SOLUTIONS file (#52.7). If this field is not set to 1, it indicates that the entry is pointed to from the DRUG file (#50). |
| .04 |  | INACTIVE DATE | This will contain the date the entry had been made inactive. |
| .05 |  | DAY (nD) or DOSE (nL) LIMIT | This is a free text field used to calculate a default value for the "STOP DATE" prompt of the order. |
| .06 |  | MED ROUTE | This is a pointer to the MEDICATION ROUTES file (#51.2). If a MED ROUTE is entered here, it will be used as the default value during order entry when this drug is selected. |
| .07 |  | SCHEDULE TYPE | This field is a set of codes and will be used as a default value when selecting this drug in order entry. |
| .08 |  | SCHEDULE | This is a free text field and will be used as a default value during order entry when this drug is selected. |
| .09 |  | SUPPLY | This is a set of codes with 1 (one) indicating that the Orderable Item is a supply. |
| 2 |  | SYNONYM (multiple) | This multiple will contain all the associated synonyms for the Pharmacy Orderable Item. |
|  | .01 | SYNONYM | A free text synonym name up to 30 characters long. |
| 3 |  | CORRESPONDING UD ITEM | If CHANGE TYPE OF ORDER FROM OERR field (#20.412) in the PHARMACY SYSTEM file (#59.7) parameter in Inpatient Mediations package is set, this field will be asked to user. This field will indicate corresponding Orderable Items to choose from and which package (UD or IV) to finish the order. |
| 4 |  | CORRESPONDING IV ITEM | If CHANGE TYPE OF ORDER FROM OERR field (#20.412) in the PHARMACY SYSTEM file (#59.7) parameter in Inpatient Mediations package is set, this field will be asked to user. This field will indicate corresponding Orderable Items to choose from and which package (UD or IV) to finish the order. |
| *5 |  | FORMULARY STATUS | This field will designate the formulary status of the Orderable Item. The non-formulary status will be displayed to the provider next to the selectable list of Orderable Item(s) during CPRS order entry (List Manager and GUI). This field is not editable. The software controls it. An Orderable Item will only be marked as non-formulary if there are no active Dispense Drugs that are formulary drugs matched to the item. |
| *6 |  | OI-DRUG TEXT ENTRY (multiple) |  |
|  | *.01 | OI-DRUG TEXT ENTRY | This is a pointer to the DRUG TEXT file (#51.7). This file contains such information as drug restrictions, guidelines and protocols to help assure that medications are being used according to formulary specifications. This information will be seen in CPRS and Pharmacy when a medication order is placed for the Pharmacy Orderable Item. New entries to the DRUG TEXT file (#51.7) must be made through the Drug Text Enter/Edit option. |
| *7 |  | PATIENT INSTRUCTIONS | The text in this field shall be presented as a default for the Patient Instructions prompt in the Outpatient Pharmacy package when entering orders, if the Dispense Drug selected is matched to this Pharmacy Orderable Item. This text will also be presented during the Outpatient Medication order entry process through CPRS, and the CPRS user can then determine whether or not these Instructions should be part of the order. For all words entered in this field, the software will check for expansions for each word in the MEDICATION INSTRUCTION file (#51) and expand the word accordingly. |
| 7.1 |  | OTHER LANGUAGE INSTRUCTIONS | The text in this field shall be presented as a default for the OTHER PATIENT INSTRUCTIONS prompt in the Outpatient Pharmacy package when entering orders, if the order being entered is for a patient who has designated a preference for another language. |
| 8 |  | NON-VA MED | This field indicates whether the Pharmacy Orderable Item is selectable as a Non-VA Med (either an herbal supplement, an over-the-counter (OTC) medication, or a prescription drug not dispensed by the VA). This field is not editable. |
| 9 |  | ASSOCIATED IMMUNIZATION | This field is added by the Immunizations Documentation by BCMA application. A mapping relationship is created between the PHARMACY ORDERABLE ITEM file (#50.7) and the pointed-to immunization so that a record can be created in the V IMMUNIZATION file (#9000010.11) corresponding to the BCMA administration of an immunization. |

## Work Flowchart for Creating and Matching Orderable Items

Below are the instructions for matching Orderable Items during the initial installation of PDM.

Steps 12 and 13 would be used to edit Orderable Items following the initial setup of PDM.

## Steps in Creating the Pharmacy Orderable Item File

The following steps to create Orderable Items must be performed prior to installing any patches to the PDM package. Following the installation of patches, these steps may not be run.  Instead, the user would access the Dispense Drug/ Orderable Item Maintenance option to create new Orderable Items.

STEP 1

The first two options in the Pharmacy Data Management menu, (Primary/VA Generic Orderable Item Report and VA Generic Orderable Item Report), should be run first. These reports will reflect what the PHARMACY ORDERABLE ITEM file (#50.7) will look like, based on which of the two methods is chosen when creating the PHARMACY ORDERABLE ITEM file (#50.7)

The Primary/VA Generic Orderable Item Report option will use your existing PRIMARY DRUG file (#50.3), as a basis of creating the new file.

The VA Generic Orderable Item Report option will use the VA Generic name from the VA GENERIC file (#50.6), as a basis for creating the new file.

There may be some instances of a drug not being matched when the report said it would be matched, but these instances should be limited to a very few.

Various factors come into play with these two methods. If the PHARMACY ORDERABLE ITEM file (#50.7) is created by VA Generic Name, then the PRIMARY DRUG file (#50.3) must be a duplication of the DRUG file (#50). In either method, the more entries in the DRUG file (#50) that are matched to the VA GENERIC file (#50.6), the more Orderable Items the software can automatically create. If the file is created by Primary/VA Generic Name, once the Primary Name portion of the option is finished, the software will also create as many entries as it can based on the VA Generic name.

One advantage of creating the PHARMACY ORDERABLE ITEM file (#50.7) by Primary Name is that all entries created via the PRIMARY DRUG file (#50.3) will have certain fields automatically populated, by being carried over from the PRIMARY DRUG file (#50.3) to the PHARMACY ORDERABLE ITEM file (#50.7). These fields are DAY (nD) or DOSE (nD), MED ROUTE, SCHEDULE TYPE, SCHEDULE, and the SYNONYM multiple.

For either method, IV Additives and IV Solutions will also have entries created for them in the PHARMACY ORDERABLE ITEM file (#50.7). Three files will point to the PHARMACY ORDERABLE ITEM file (#50.7); they are the DRUG file (#50), the IV ADDITIVES file (#52.6), and the IV SOLUTIONS file (#52.7).

STEP 2

The third option in the Pharmacy Data Management menu, Create Pharmacy Orderable Items, is the option that actually creates the entries in the PHARMACY ORDERABLE ITEM file (#50.7), and creates the pointers in the files mentioned in step 1. Based on the reports generated in step 1, the new file can be created by using either of the two methods.

STEP 3

The Manually Match Dispense Drugs option should be run next. This option will step through all the remaining entries in the IV ADDITIVES file (#52.6), the IV SOLUTIONS file (#52.7), and the DRUG file (#50), and ask the user to match any unmatched drugs to an entry in the PHARMACY ORDERABLE ITEM file (#50.7). The user will be required to match all Additives and Solutions with valid pointers to the DRUG file (#50), and all entries in the DRUG file (#50) that are marked with an I, O, or U in the APPLICATION PACKAGES’ USE field (#63) which are active or have an active date of a year or less.

Each time this option is run, upon exit of the option, a check will be done to see if there are any drugs that are still not matched that should be matched according to the rules mentioned above. If there are, a message will be given stating this and reminding the user that someone will have to return to this option and continue matching drugs until they are all matched. If upon exit, all the necessary drugs are matched, a message will be given, and a job will be queued to do a final check on the file for any invalid data. Upon completion of that queued job, a mail message will be sent to the user who completed the matching process stating that everything has been completed.

The other options in the Pharmacy Data Management menu provide methods of maintaining the PHARMACY ORDERABLE ITEM file (#50.7) and the pointer relationships of the other files.

## File List

The following PDM files are exported with the PDM package.

File#       NAME          	UPDATE 	DATA COMES    USER

DD	WITH FILE   OVERRIDE

50          DRUG 	FULL	NO

50.4        DRUG ELECTROLYTES 	FULL	NO

50.606      DOSAGE FORM                  	FULL	YES (MERGE)	NO

50.7        PHARMACY ORDERABLE ITEM 	FULL	NO

51          MEDICATION INSTRUCTION        	FULL	NO

51.1        ADMINISTRATION SCHEDULE      	FULL	YES (MERGE)	YES

51.2        MEDICATION ROUTES           	FULL	YES (MERGE)	YES

51.5        ORDER UNIT 	FULL	NO

51.7        DRUG TEXT	FULL	YES	YES

(OVERWRITE)

52.6        IV ADDITIVES 	FULL	NO

52.7        IV SOLUTIONS 	FULL	NO

54          RX CONSULT 	FULL (SCREEN)	NO

55          PHARMACY PATIENT (Partial DD)	PARTIAL	NO

59.7        PHARMACY SYSTEM 	FULL	NO

The following non-PDM files are exported with the PDM package.

File#       NAME          	UPDATE 	DATA COMES    USER

DD	WITH FILE   OVERRIDE

200         NEW PERSON (Partial DD)	PARTIAL	NO

9009032.3   APSP INTERVENTION TYPE      	FULL	YES		NO

(OVERWRITE)

9009032.4   APSP INTERVENTION 	FULL	NO

9009032.5   APSP INTERVENTION RECOMMENDATION 	FULL	YES	NO

(OVERWRITE)

(This page included for two-sided copying.)

File Descriptions

File descriptions can be viewed by using the VA FileMan Print File Entries option. If files are out of sync they will need to be entered individually at the "START WITH NUMBER" prompt.

Select OPTION: PRINT FILE ENTRIES

OUTPUT FROM WHAT FILE: PHARMACY ORDERABLE ITEM// FILE

1   FILE                             (490 entries)

2   FILEGRAM ERROR LOG               (0 entries)

3   FILEGRAM HISTORY                 (0 entries)

CHOOSE 1-3: 1

SORT BY: NAME// .001  NUMBER

START WITH NUMBER: FIRST// 50.7

GO TO NUMBER: LAST// 50.7

WITHIN NUMBER, SORT BY:

FIRST PRINT ATTRIBUTE: 1  GLOBAL NAME

THEN PRINT ATTRIBUTE: .01  NAME

THEN PRINT ATTRIBUTE: DESCRIPTION      (word-processing)

THEN PRINT ATTRIBUTE: <Enter>

*************************

Heading (S/C): FILE LIST// <Enter>

START AT PAGE: 1// <Enter>

DEVICE: <Enter>

FILE LIST                                      JUL 10,2001  13:18    PAGE 1

GLOBAL NAME     NAME

DESCRIPTION

------------------------------------------------------------------------------

NUMBER: 50.7

^PS(50.7,       PHARMACY ORDERABLE ITEM

Order Entry name for items that can be ordered in the Pharmacy package.

(This page included for two-sided copying.)

## Option Descriptions

The option descriptions below were retrieved from VA FileMan and provide the PDM options following the initial installation of the PDM package.

------------------------------------------------------------------

#### PSS MGR Pharmacy Data Management

This menu contains the options necessary to build and maintain the PHARMACY ORDERABLE ITEM file (#50.7), and to also build and maintain the Med. Route/Instructions table.

Type: menu

Item:PSS PRIMARY/VA GENERIC REPORT

Item:PSS VA GENERIC DRUG REPORT

Item:PSS CREATE ORDERABLE ITEMS

Item:PSS PRE-RELEASE MANUAL MATCH

Item:PSS ORDERABLE ITEM STATUS

Item:PSSXX MARK

Item:PSS DRUG ENTER/EDIT

Item:PSS DRG INTER MANAGMENT

Item:PSS INTERACTION LOCAL ADD

Item:PSS INTERACTION SEVERITY

Item:PSSJI ELECTROLYTE FILE

Item:PSS LOOK

Item:PSS MED. ROUTE/INSTRUCTIONS

Item:PSSJU MI

Item:PSS ORDERABLE ITEM MANAGEMENT

Item:PSS EDIT ORDERABLE ITEMS

Item:PSS MAINTAIN ORDERABLE ITEMS

Item:PSS ADDITIVE/SOLUTIONS

Item:PSS ORDERABLE ITEM REPORT

Item:PSS PRIMARY DRUG EDIT

Item:PSS SYS EDIT

Item:PSS SCHEDULE EDIT

----------------------------------------------------------------

PSS PRIMARY/VA GENERIC REPORT
Primary/VA Generic Orderable Item Report

This option provides a report of potential Pharmacy Orderable Items based on creating your PHARMACY ORDERABLE ITEM file (#50.7) by Primary Drug name, then VA Generic Name. This report can be run to show drugs that can be auto-created, or be run to show drugs that can’t be auto-created.

Type: run routine   Routine: PRI^PSSMATCH

----------------------------------------------------------------

PSS VA GENERIC DRUG REPORT
VA Generic Orderable Item Report

This option provides a report of potential Pharmacy Orderable Items based on creating your PHARMACY ORDERABLE ITEM file (#50.7) by VA Generic name only. This report can be run to show drugs that can be auto-created, or run to show drugs that can’t be auto-created.

Type:run routine   Routine: VA^PSSMATCH

---------------------------------------------------------------

PSS CREATE ORDERABLE ITEMS
Create Pharmacy Orderable Items

This option is used to populate the PHARMACY ORDERABLE ITEM file (#50.7). It will also match entries in the IV ADDITIVES file (#52.6), the IV SOLUTIONS file (#52.7), and the DRUG file (#50) to the new PHARMACY ORDERABLE ITEM file (#50.7). This option can only be run one time. It will queue a background job that builds a large TMP global that will be used to create the new file. Do not run this option until you are sure of the method that you want to create the file by. See the Implementation and Maintenance section of this manual, (Steps In Creating Orderable Item File), for more information.

Type: run routine     Routine: PSSCREAT

----------------------------------------------------------------

----------------------------------------------------------------

PSS PRE-RELEASE MANUAL MATCH
Manually Match Dispense Drugs

This option is used to manually match IV Additives, IV Solutions, and Dispense Drugs to Pharmacy Orderable Items.

This option must be invoked after the PSS CREATE ORDERABLE ITEMS option has been run to completion. This option will force the matching of all Additives and Solutions with Dispense Drug pointers, and all Dispense Drugs marked with an I, O, or U in the APPLICATION PACKAGES’ USE field (#63). Every time this option is exited, a check will be done for any unmatched drugs, and an appropriate message will be given to the user. When all drugs are matched that should be matched, a job will be queued that checks for any invalid data. At the completion of this job, a mail message will be sent to the user who completed the matching process. At this time, the pre-release work is finished, and installation of Outpatient Pharmacy V. 7.0 and Inpatient Medications V. 5.0 will be allowed.

Type: run routine   Routine: PSSPOIM2

------------------------------------------------------------------

#### PSS ORDERABLE ITEM STATUS
Orderable Item Matching Status

This option allows the user to check the status of the manual matching of Orderable Items.

Type: run routine       PSSOICT

------------------------------------------------------------------

PSSXX MARK
CMOP Mark/Unmark (Single Drug)

This option allows the user to mark/unmark a single drug for transmission to the CMOP.

Type: run routine      PSSMARK

-----------------------------------------------------------------

PSS DRUG ENTER/EDIT
Drug Enter/Edit

This option allows the user to edit fields for ALL pharmacy packages if they possess the proper package key. It also will allow the user to match to NDF and Orderable Item.

Type:run routine   ROUTINE: PSSDEE

-----------------------------------------------------------------

PSS DRG INTER MANAGEMENT
Drug Interaction Management

This is the sub-menu driver for drug interaction maintenance.

Type:menu

-----------------------------------------------------------------

#### PSS INTERACTION LOCAL ADD
Enter/Edit Local Drug Interaction

This option allows pharmacy sites to add local drug ingredient interactions to the DRUG/DRUG INTERACTION file or edit only local drug interactions already in the file. The national entries in the DRUG/DRUG INTERACTION file cannot be edited.

This information displays on a screen or prints on a label whenever a pharmacist fills a prescription.

Type:action   ENTRY ACTION: D ADD^PSSHELP

----------------------------------------------------------------

-----------------------------------------------------------------

PSS INTERACTION SEVERITY
Edit Drug Interaction Severity

With this option the pharmacy manager can change the drug interaction severity in the DRUG/DRUG INTERACTION file from significant to critical only. Severity cannot be changed from critical to significant.

Type:action  	ENTRY ACTION: D CRI^PSSHELP

----------------------------------------------------------------

PSSJI ELECTROYLYTE FILE
Electrolyte File (IV)

This option will allow you to alter the contents of the DRUG ELECTORYLYTES file (#50.4). This is the file that is pointed to by the ELECTROLYTE field in both the IV ADDITIVES (#52.6) and IV SOLUTIONS (#52.7) files.

Type:run routine   Routine: ELECTRO^PSSIVDRG

-----------------------------------------------------------------

#### PSS LOOK

Lookup into Dispense Drug File

This option provides a report of all information regarding the dispense drug.

Type:run routine   ROUTINE: PSSLOOK

-----------------------------------------------------------------

PSS MED. ROUTE/INSTRUCTIONS
Med. Route/Instructions Table Maintenance

This option is used to maintain the table of medication routes that will be displayed as defaults in OE/RR, based on dosage form.

Type: run routine     ROUTINE: ENMR^PSSFILED

-----------------------------------------------------------------

PSSJU MI
Medication Instruction File Add/Edit

The UNIT DOSE package contains a field called SPECIAL INSTRUCTIONS. This field utilizes the abbreviations and expansions of the MEDICATION INSTRUCTION file when printing on various reports. This option allow the user to enter and edit abbreviations and expansions in the MEDICATION INSTRUCTION file and to "flag" those entries for use by the INPATIENT packages only, OUTPATIENT package only, or by both.

Type:run routine   ROUTINE: ENMI^PSSFILED

----------------------------------------------------------------

PSS ORDERABLE ITEM MANAGEMENT
Orderable Item Management

This is the sub-menu driver for Orderable Item maintenance.

Type:menu

---------------------------------------------------------------

#### PSS EDIT ORDERABLE ITEMS
Edit Orderable Items

This option is used to edit entries in the PHARMACY ORDERABLE ITEM

file (#50.7). All fields except the DOSAGE FORM and IV FLAG fields can be edited.

Type: run routine     Routine: PSSPOIMO

---------------------------------------------------------------

------------------------------------------------------------------

PSS MAINTAIN ORDERABLE ITEMS
Dispense Drug/Orderable Item Maintenance

This option is for maintaining the relationship between Dispense Drugs and Orderable Items.

This option is used to maintain the relationship between Dispense Drugs and entries in the PHARMACY ORDERABLE ITEM file (#50.7). The Dispense Drug will be prompted for, and then the user can match it to a Pharmacy Orderable Item, or re-match it to a different Pharmacy Orderable Item, depending on whether the Dispense Drug chosen has already been matched. This option is different from the PSS PRE-RELEASE MANUAL MATCH option, where this prompts for Dispense Drug, and this option will remain on the Pharmacy Data Management menu for maintenance purposes. The PSS PRE-RELEASE MANUAL MATCH option is only for the initial set-up of the PHARMACY ORDERABLE ITEM file (#50.7), and should be completed prior to the installations of Inpatient Medications V. 5.0 and Outpatient Pharmacy V. 7.0.

Type: run routine     Routine: PSSPOIMN

---------------------------------------------------------------

#### PSS ADDITIVE/SOLUTIONS
Additive/Solutions, Orderable Items

This option is for maintaining the links between entries in the IV ADDITIVES file (#52.6) and the PHARMACY ORDERABLE ITEM file (#50.7),and also between the IV SOLUTIONS file (#52.7) and the PHARMACY ORDERABLE ITEM file (#50.7). Once an Additive or Solution is chosen, the user can then match to an Orderable Item, or re-match to different Orderable Item, depending on whether the Additive or Solution chosen has already been matched.

Type: run routine     Routine: PSSADDIT

----------------------------------------------------------------

PSS ORDERABLE ITEM REPORT
Orderable Item Report

This option prints a report that is broken up into three parts. The first part shows all Pharmacy Orderable Items that are linked to IV Additives. The second part shows all Pharmacy Orderable Items linked to IV Solutions. The third part shows all Pharmacy Orderable Items linked to Dispense Drugs.

Type: run routine   Routine: PSSPOIKA

------------------------------------------------------------------

PSS PRIMARY DRUG EDIT
Primary Drug Edit

Allows the user to create and edit entries in the PRIMARY DRUG file.

Type: run routine     Routine: ENPDE^PSSFIL

-----------------------------------------------------------------

PSS SYS EDIT
Pharmacy System Parameters Edit

This will be the edit IV Identifier field that will be displayed with IV Orderable Items. The IV Identifier will be set to “IV” when the package is initially installed.

The Change Type of Order From OERR field is a site parameter that will allow the pharmacist to change the type of order from what is received from OERR. If this field is set to yes, it will be possible to change the order type on orders where the Orderable Item has data in the CORRESPONDING IV ITEM field for unit dose orders or data in the CORRESPONDING UD ITEM for IV orders.

Type:run routine   Routine: PSSYSP

-----------------------------------------------------------------

#### PSS SCHEDULE EDIT
Standard Schedule Edit

This option allows the user to edit the set of times associated with the standard dosage administration schedules. This may be used to define the outpatient expansion to be used when the schedule is entered for an outpatient medication order.

Type:run routine   Routine: ENPSJSE^PSSJEEU

The option descriptions below were retrieved from VA FileMan and reflect the PDM options following the installation of patch PSS*1*38.

----------------------------------------------------------------

NAME: PSS MGR

MENU TEXT: Pharmacy Data Management

TYPE: menu

PACKAGE: PHARMACY DATA MANAGEMENT

DESCRIPTION:   This menu contains options pertaining to maintaining pharmacy

data files, creating Pharmacy Orderable Items, and the Medication Route/

Instructions table among other assorted functions.

ITEM: PSSXX MARK

ITEM: PSS DOSAGES MANAGEMENT

ITEM: PSS DRUG ENTER/EDIT

ITEM: PSS DRG INTER MANAGEMENT

ITEM: PSSJI ELECTROLYTE FILE

ITEM: PSS LOOK

ITEM: PSSJU MI

ITEM: PSS MEDICATION ROUTES EDIT

ITEM: PSS ORDERABLE ITEM MANAGEMENT

ITEM: PSS ORDERABLE ITEM REPORT

ITEM: PSSNFI

ITEM: PSS EDIT TEXT

ITEM: PSS DRUG TEXT FILE REPORT

ITEM: PSS SYS EDIT

ITEM: PSS SCHEDULE EDIT

ITEM: PSS SYNONYM EDIT

----------------------------------------------------------------

NAME: PSS DOSAGES MANAGEMENT

MENU TEXT: Dosages

TYPE: menu

DESCRIPTION:   This menu option contains options that control the editing of

dosages.

ITEM: PSS DOSAGE CONVERSION

ITEM: PSS DOSAGE FORM EDIT

ITEM: PSS EDIT DOSAGES

ITEM: PSS COMMON DOSAGES

ITEM: PSS DOSE FORM/NOUN REPORT

ITEM: PSS DOSAGE REVIEW REPORT

----------------------------------------------------------------

NAME: PSS DRG INTER MANAGEMENT

MENU TEXT: Drug Interaction Management

TYPE: menu

DESCRIPTION:   This is the sub-menu driver for drug interaction maintenance.

ITEM: PSS INTERACTION LOCAL ADD

ITEM: PSS REPORT LOCAL INTERACTIONS

----------------------------------------------------------------

#### NAME: PSS ORDERABLE ITEM MANAGEMENT

MENU TEXT: Orderable Item Management

TYPE: menu

DESCRIPTION:   This is the sub-menu driver for Orderable Item maintenance.

ITEM: PSS MAINTAIN ORDERABLE ITEMS

ITEM: PSS EDIT ORDERABLE ITEMS

ITEM: PSS ORDERABLE ITEM DOSAGES

ITEM: PSS INSTRUCTIONS/ITEMS REPORT

----------------------------------------------------------------

NAME: PSSXX MARK

MENU TEXT: CMOP Mark/Unmark (Single drug)

TYPE: run routine

LOCK: PSXCMOPMGR

DESCRIPTION:   This option allows the user to mark/unmark a single drug for

transmission to the CMOP.

ROUTINE: PSSMARK

UPPERCASE MENU TEXT: CMOP MARK/UNMARK (SINGLE DRUG)

----------------------------------------------------------------

NAME: PSS DOSAGE CONVERSION

MENU TEXT: Auto Create Dosages

TYPE: run routine

DESCRIPTION:   This option is used to populate the Possible Dose fields and

Local Possible Dose fields in the DRUG file.

ROUTINE: PSSDOSCR

UPPERCASE MENU TEXT: AUTO CREATE DOSAGES

----------------------------------------------------------------

NAME: PSS DOSAGE FORM EDIT

MENU TEXT: Dosage Form File Enter/Edit

TYPE: run routine

DESCRIPTION:   This option provides the ability to edit data in the DOSAGE

FORM file (#50.606).

ROUTINE: DF^PSSDFEE

UPPERCASE MENU TEXT: DOSAGE FORM FILE ENTER/EDIT

----------------------------------------------------------------

NAME: PSS EDIT DOSAGES

MENU TEXT: Enter/Edit Dosages

TYPE: run routine

DESCRIPTION:   This option allows you to edit Possible Dose fields and Local

Possible Dose fields for a selected dispense drug.

ROUTINE: DOS^PSSDOSER

UPPERCASE MENU TEXT: ENTER/EDIT DOSAGES

----------------------------------------------------------------

NAME: PSS COMMON DOSAGES

MENU TEXT: Most Common Dosages Report

TYPE: run routine

DESCRIPTION:   This report displays the most common dosages administered over

a specified time period for Unit Dose orders.

ROUTINE: EN^PSSCOMMN

UPPERCASE MENU TEXT: MOST COMMON DOSAGES REPORT

----------------------------------------------------------------

NAME: PSS DOSE FORM/NOUN REPORT

MENU TEXT: Noun/Dosage Form Report

TYPE: run routine

DESCRIPTION:   This report displays the Dosage Forms, along with their

associated Nouns and Package Use field. It also displays the Local Possible

Dose based on the nouns and instructions.

ROUTINE: PSSNOUNR

UPPERCASE MENU TEXT: NOUN/DOSAGE FORM REPORT

----------------------------------------------------------------

NAME: PSS DOSAGE REVIEW REPORT

MENU TEXT: Review Dosages Report

TYPE: run routine

DESCRIPTION:   This report shows the possible doses and local possible doses

for selected dispense drugs.

ROUTINE: PSSDOSRP

UPPERCASE MENU TEXT: REVIEW DOSAGES REPORT

----------------------------------------------------------------

NAME: PSS DRUG ENTER/EDIT

MENU TEXT: Drug Enter/Edit

TYPE: run routine

DESCRIPTION:   This option allows the user to edit fields for ALL pharmacy

packages if they possess the proper package key. It also will allow the user

to match to NDF and Orderable Item.

ROUTINE: PSSDEE

UPPERCASE MENU TEXT: DRUG ENTER/EDIT

----------------------------------------------------------------

NAME: PSS INTERACTION LOCAL ADD

MENU TEXT: Enter/Edit Local Drug Interaction

TYPE: run routine

PACKAGE: PHARMACY DATA MANAGEMENT

DESCRIPTION:   This option allows sites to add their local drug ingredient

interactions.

ROUTINE: PSSDINT

UPPERCASE MENU TEXT: ENTER/EDIT LOCAL DRUG INTERACTION

----------------------------------------------------------------

NAME: PSS REPORT LOCAL INTERACTIONS

MENU TEXT: Report of Locally Entered Interactions

TYPE: run routine

DESCRIPTION:   This option allows sites to print a report of locally entered

drug interactions and their severity.

ROUTINE: PSSDRINT

UPPERCASE MENU TEXT: REPORT OF LOCALLY ENTERED INTERACTIONS

----------------------------------------------------------------

#### NAME: PSSJI ELECTROLYTE FILE

MENU TEXT: Electrolyte File (IV)

TYPE: run routine

DESCRIPTION:

This option will allow you to alter the contents of the DRUG ELECTROLYTES

file (50.4). This is the file that is pointed to by the ELECTROLYTES sub-file

in the IV ADDITIVES (52.6) and IV SOLUTIONS (#52.7) files.

EXIT ACTION: K I1

ROUTINE: ELECTRO^PSSVIDRG

UPPERCASE MENU TEXT: ELECTROLYTE FILE (IV)

----------------------------------------------------------------

NAME: PSS LOOK

MENU TEXT: Lookup into Dispense Drug File

TYPE: run routine

DESCRIPTION:   This option provides a lookup into DISPENSE DRUG file. It

displays fields commonly edited.

ROUTINE: PSSLOOK

UPPERCASE MENU TEXT: LOOKUP INTO DISPENSE DRUG FILE

----------------------------------------------------------------

NAME: PSSJU MI

MENU TEXT: Medication Instruction File Add/Edit

TYPE: run routine

DESCRIPTION:   The UNIT DOSE package contains a field called SPECIAL

INSTRUCTIONS. This field utilizes the abbreviations and expansions of the

MEDICATION INSTRUCTION file when printing on various reports. This option

allow the user to enter and edit abbreviations and expansions in the

MEDICATION INSTRUCTION file and to "flag" those entries for use by the

INPATIENT packages only, OUTPATIENT package only, or by both.

ROUTINE: ENMI^PSSFILED

UPPERCASE MENU TEXT: MEDICATION INSTRUCTION FILE ADD/EDIT

----------------------------------------------------------------

NAME: PSS MEDICATION ROUTES EDIT

MENU TEXT: Medication Route File Enter/Edit

TYPE: run routine

DESCRIPTION:   This option provides the ability to edit data in the MEDICATION

ROUTES file (#51.2).

ROUTINE: MR^PSSDFEE

UPPERCASE MENU TEXT: MEDICATION ROUTE FILE ENTER/EDIT

----------------------------------------------------------------

NAME: PSS EDIT ORDERABLE ITEMS

MENU TEXT: Edit Orderable Items

TYPE: run routine

DESCRIPTION:   This option is for editing Pharmacy Orderable Items.

ROUTINE: PSSPOIMO

UPPERCASE MENU TEXT: EDIT ORDERABLE ITEMS

----------------------------------------------------------------

NAME: PSS MAINTAIN ORDERABLE ITEMS

MENU TEXT: Dispense Drug/Orderable Item Maintenance

TYPE: run routine

DESCRIPTION:   This option is for maintaining the relationship between

Dispense Drugs and Orderable Items.

ROUTINE: PSSPOIMN

UPPERCASE MENU TEXT: DISPENSE DRUG/ORDERABLE ITEM MAINTENANCE

----------------------------------------------------------------

NAME: PSS ORDERABLE ITEM DOSAGES

MENU TEXT: Orderable Item/Dosages Report

TYPE: run routine

DESCRIPTION:   This option prints a report that displays Inpatient and

Outpatient Dosages for each Pharmacy Orderable Item. These are the dosages

that will display for selection through Computerized Patient Record System

(CPRS) when an Orderable Item is selected for a medication order.

ROUTINE: EN^PSSOIDOS

UPPERCASE MENU TEXT: ORDERABLE ITEM/DOSAGES REPORT

----------------------------------------------------------------

NAME: PSS INSTRUCTIONS/ITEMS REPORT

MENU TEXT: Patient Instructions Report

TYPE: run routine

DESCRIPTION:   This option prints a report that displays Pharmacy Orderable

Items, along with the expanded Patient Instructions for each Orderable Item.

These Patient Instructions are used as default values for Outpatient orders

entered through Computerized Patient Record System (CPRS) and Outpatient

Pharmacy.

ROUTINE: EN^PSSPNSRP

UPPERCASE MENU TEXT: PATIENT INSTRUCTIONS REPORT

----------------------------------------------------------------

NAME: PSS ORDERABLE ITEM REPORT

MENU TEXT: Orderable Item Report

TYPE: run routine

DESCRIPTION:   This option lists items from your Pharmacy Orderable Item file,

along with the associated Dispense Drugs.

ROUTINE: PSSPOIKA

UPPERCASE MENU TEXT: ORDERABLE ITEM REPORT

----------------------------------------------------------------

NAME: PSSNFI

MENU TEXT: Formulary Information Report

TYPE: run routine

DESCRIPTION:   This option provides a listing of pertinent pharmacy formulary

information.

ROUTINE: PSSNFI

UPPERCASE MENU TEXT: FORMULARY INFORMATION REPORT

----------------------------------------------------------------

NAME: PSS EDIT TEXT

MENU TEXT: Drug Text Enter/Edit

TYPE: run routine

DESCRIPTION:   This option enables you to edit entries in the DRUG TEXT file.

This file contains drug information, restrictions, and guidelines, etc.

ROUTINE: PSSTXT

UPPERCASE MENU TEXT: DRUG TEXT ENTER/EDIT

----------------------------------------------------------------

NAME: PSS SYS EDIT

MENU TEXT: Pharmacy System Parameters Edit

TYPE: run routine

DESCRIPTION:   This option allows the user to edit the Pharmacy System

parameters used in Pharmacy Data Management.

ROUTINE: PSSYSP

UPPERCASE MENU TEXT: PHARMACY SYSTEM PARAMETERS EDIT

----------------------------------------------------------------

NAME: PSS SCHEDULE EDIT

MENU TEXT: Standard Schedule Edit

TYPE: run routine

DESCRIPTION:   Allows user to edit the set of times associated with the

standard dosage administration schedules (Q3H,Q8H,TID, etc.).  This may be

used to define the outpatient expansion to be used when the schedule is

entered for an outpatient medication order.

ROUTINE: ENPSJSE^PSSJEEU

UPPERCASE MENU TEXT: STANDARD SCHEDULE EDIT

----------------------------------------------------------------

NAME: PSS SYNONYM EDIT

MENU TEXT: Synonym Enter/Edit

TYPE: run routine

DESCRIPTION:   The option provides easy access to update the synonym

information for an entry in the local DRUG file.

ROUTINE: PSSSEE

UPPERCASE MENU TEXT: SYNONYM ENTER/EDIT

The option description below was retrieved from VA FileMan and reflects the new option added to PDM following the installation of patch PSS*1*55.

----------------------------------------------------------------

NAME: PSS DRUG TEXT FILE REPORT

MENU TEXT: Drug Text File Report

TYPE: run routine

DESCRIPTION:   This option will display the Drug Text Name, Synonyms, Drug

Text, Inactive date if inactive, and list all drug and pharmacy orderable

items linked to it.  The report is printable for one or all entries.

ROUTINE: PSSDTR

UPPERCASE MENU TEXT: DRUG TEXT FILE REPORT

The option description below was retrieved from VA FileMan and reflects the new option added to PDM following the installation of patch PSS*1*47.  Other Language Translation Setup [PSS OTHER LANGUAGE SETUP] option is a stand-alone option that must be assigned to the person(s) responsible for maintaining it.

----------------------------------------------------------------

NAME: PSS OTHER LANGUAGE SETUP

MENU TEXT: Other Language Translation Setup

TYPE: action

PACKAGE: PHARMACY DATA MANAGEMENT

DESCRIPTION:   This option can be used to enter other language translations

that will print on the outpatient pharmacy Rx labels.

ENTRY ACTION: D OTHLAN^PSSJEEU

UPPERCASE MENU TEXT: OTHER LANGUAGE TRANSLATION SET

The option description below was retrieved from VA FileMan and reflects the new option added to PDM following the installation of PSS*1*82.

----------------------------------------------------------------

NAME: PSS MASTER FILE ALL

MENU TEXT: Send Entire Drug File to External Interface

TYPE: run routine

DESCRIPTION:   This option will send the entire DRUG file (#50) to an external

interface through HL7 messaging.

ROUTINE: PSSMSTR

UPPERCASE MENU TEXT: SEND ENTIRE DRUG FILE TO EXTER

The two option descriptions below were retrieved from VA FileMan and reflect the new options added to PDM following the installation of PSS*1*87.

----------------------------------------------------------------

NAME: PSS WARNING BUILDER

MENU TEXT: Warning Builder

TYPE: run routine

DESCRIPTION:   This option will allow you to define a custom warning label

list containing entries from both the new warning label source and the old Rx

Consult file entries.

ROUTINE: PSSWRNB

UPPERCASE MENU TEXT: WARNING BUILDER

----------------------------------------------------------------

NAME: PSS WARNING MAPPING

MENU TEXT: Warning Mapping

TYPE: run routine

DESCRIPTION:   This option is used to match an entry from the old Rx Consult

file to the new commercial data source warning file to aid in using the

Warning Builder (to identify local warnings that do not have an equivalent

entry in the new commercial data source).

The user can also enter a Spanish translation for an Rx Consult file entry, if

desired, but whenever possible, the new commercial data source's warnings

(English or Spanish depending on the patient setting) should be used.

ROUTINE: EDIT^PSSWMAP

UPPERCASE MENU TEXT: WARNING MAPPING

The following options were retrieved from VA FileMan and reflect the new options added to PDM following the installation of PSS*1*129.

----------------------------------------------------------------

This option is duplicated on the ENHANCED ORDER CHECKS SETUP menu

NAME: PSS LOCAL POSSIBLE DOSAGES

MENU TEXT: Local Possible Dosages Report

TYPE: run routine

DESCRIPTION:   This option provides a report that displays entries from the

LOCAL POSSIBLE DOSAGE (#50.0904) Subfile of the DRUG (#50) File with missing

data in DOSE UNIT (#4) Field and the NUMERIC DOSE (#5) Field. This data needs

to be populated if Dosage checks are to be performed, when that Local Possible

Dosage is selected for an order.

ROUTINE: EN^PSSLDOSE

UPPERCASE MENU TEXT: LOCAL POSSIBLE DOSAGES REPORT

----------------------------------------------------------------

NAME: PSS DOSE UNIT REQUEST

This option is duplicated on the ENHANCED ORDER CHECKS SETUP menu

MENU TEXT: Request Change to Dose Unit

TYPE: run routine

DESCRIPTION:   This option enables a request to be made to have a new entry

added, or a current entry changed in the DOSE UNITS (#51.24) File.

ROUTINE: DOSE^PSSMEDRQ

UPPERCASE MENU TEXT: REQUEST CHANGE TO DOSE UNIT

----------------------------------------------------------------

NAME: PSS MED INSTRUCTION MANAGEMENT

MENU TEXT: Medication Instruction Management

TYPE: menu

DESCRIPTION:  The Sub-Menu contains options related to the MEDICATION

INSTRUCTION (#51) File.

ITEM: PSSJU

ITEM: PSS MED INSTRUCTION REPORT

----------------------------------------------------------------

NAME: PSS MED INSTRUCTION REPORT

MENU TEXT: Medication Instruction File Report

TYPE: run routine

DESCRIPTION:   This option provides a report of entries from the MEDICATION

INSTRUCTION (#51) File that shows whether or not data has been entered in the

FREQUENCY (IN MINUTES) (#31) Field. To perform Dosage checks on medication

orders, a frequency must be derived from the Schedule of the order.

ROUTINE: EN^PSSMIRPT

UPPERCASE MENU TEXT: MEDICATION INSTRUCTION FILE RE

----------------------------------------------------------------

NAME: PSS MEDICATION ROUTES MGMT

MENU TEXT: Medication Routes Management

TYPE: menu

DESCRIPTION:  This Sub-Menu contains options related to Medication Routes in

both the MEDICATION ROUTES (#51.2) File and the STANDARD MEDICATION ROUTES

(#51.23) File.

ITEM: PSS MEDICATION ROUTES EDIT

These four options are duplicated on the ENHANCED ORDER CHECKS SETUP menu

ITEM: PSS MED ROUTE MAPPING REPORT

ITEM: PSS MED ROUTE MAPPING CHANGES

ITEM: PSS MEDICATION ROUTE REQUEST

----------------------------------------------------------------

NAME: PSS MEDICATION ROUTES EDIT

MENU TEXT: Medication Route File Enter/Edit

TYPE: run routine

DESCRIPTION:   This option provides the ability to edit data for entries in

the MEDICATION ROUTES (#51.2) File.

ROUTINE: MR^PSSDFEE

UPPERCASE MENU TEXT: MEDICATION ROUTE FILE ENTER/ED

----------------------------------------------------------------

NAME: PSS MED ROUTE MAPPING REPORT

MENU TEXT: Medication Route Mapping Report

TYPE: run routine

DESCRIPTION:   This option provides a report that shows the status of the

current mapping from the Medication Routes (#51.2) File to the Standard

Medication Routes (#51.23) File.

ROUTINE: REP^PSSMEDRT

UPPERCASE MENU TEXT: MEDICATION ROUTE MAPPING REPOR

----------------------------------------------------------------

NAME: PSS MED ROUTE MAPPING CHANGES

MENU TEXT: Medication Route Mapping History Report

TYPE: run routine

DESCRIPTION:   This option will provide a report that shows all of the mapping

changes between entries from the MEDICATION ROUTES (#51.2) File to entries in

the STANDARD MEDICATION ROUTES (#51.23) File.

ROUTINE: DRIVER^PSSMEDCH

UPPERCASE MENU TEXT: MEDICATION ROUTE MAPPING HISTO

----------------------------------------------------------------

NAME: PSS MEDICATION ROUTE REQUEST

MENU TEXT: Request Change to Standard Medication Route

TYPE: run routine

DESCRIPTION:   This option enables a request to be made to have a new entry

added, or a current entry changed in the STANDARD MEDICATION ROUTES (#51.23)

File.

ROUTINE: REQ^PSSMEDRQ

UPPERCASE MENU TEXT: REQUEST CHANGE TO STANDARD MED

----------------------------------------------------------------

NAME: PSS DRUG TEXT MANAGEMENT

MENU TEXT: Drug Text Management

TYPE: menu

DESCRIPTION:   This Sub-Menu contains options concerning Drug Text.

ITEM: PSS EDIT TEXT                     DISPLAY ORDER: 1

ITEM: PSS DRUG TEXT FILE REPORT         DISPLAY ORDER: 2

----------------------------------------------------------------

NAME: PSS SCHEDULE MANAGEMENT

MENU TEXT: Standard Schedule Management

TYPE: menu

DESCRIPTION:  This Sub-Menu contains options needed for Schedule maintenance.

ITEM: PSS SCHEDULE EDIT

ITEM: PSS SCHEDULE REPORT

----------------------------------------------------------------

NAME: PSS SCHEDULE REPORT

MENU TEXT: Administration Schedule File Report

TYPE: run routine

DESCRIPTION:   This option provides a report of entries from the

ADMINISTRATION SCHEDULE (#51.1) File that shows whether or not data has been

entered in the FREQUENCY (IN MINUTES) (#2) Field for the entries. To perform

Dosage checks on medication orders, a frequency must be derived from the

Schedule of the order.

ROUTINE: EN^PSSSCHRP

UPPERCASE MENU TEXT: ADMINISTRATION SCHEDULE FILE R

----------------------------------------------------------------

NAME: PSS ENHANCED ORDER CHECKS

MENU TEXT: Enhanced Order Checks Setup Menu

TYPE: menu

DESCRIPTION:   This menu option contains the options needed for Pharmacy to do

the appropriate setup needed to install Pharmacy Re-engineering version 0.5.

Version 0.5 mainly deals with the Enhanced Order Checks.

ITEM: PSS MED ROUTES INITIAL MAPPING

ITEM: PSS MAP ONE MED ROUTE

ITEM: PSS MED ROUTE MAPPING REPORT

These four options are duplicated on the MEDICATION ROUTES MANAGEMENT menu

ITEM: PSS MEDICATION ROUTES EDIT

ITEM: PSS MED ROUTE MAPPING CHANGES

ITEM: PSS MEDICATION ROUTE REQUEST

ITEM: PSS LOCAL DOSAGES EDIT ALL

ITEM: PSS LOCAL DOSAGES EDIT

<< This option is duplicated on the DOSAGES menu

ITEM: PSS LOCAL POSSIBLE DOSAGES

ITEM: PSS STRENGTH MISMATCH

ITEM: PSS EDIT DOSAGES

<< This option is duplicated on the DOSAGES menu

ITEM: PSS DOSE UNIT REQUEST

ITEM: PSS MARK PREMIX SOLUTIONS

ITEM: PSS IV SOLUTION REPORT

ITEM: PSS SCHEDULE REPORT

ITEM: PSS MED INSTRUCTION REPORT

----------------------------------------------------------------

NAME: PSS MED ROUTES INITIAL MAPPING

MENU TEXT: Find Unmapped Local Medication Routes

PACKAGE: PHARMACY DATA MANAGEMENT

DESCRIPTION:   This option will loop through all entries in the MEDICATION

ROUTES (#51.2) File and find entries that are marked for 'ALL PACKAGES' in the

PACKAGE USE (#3) Field that are not mapped to an entry in the STANDARD

MEDICATION ROUTES (#51.23) File, and prompt for the mapping.

ROUTINE: MATCH^PSSMEDRT

UPPERCASE MENU TEXT: FIND UNMAPPED LOCAL MEDICATION

----------------------------------------------------------------

NAME: PSS MAP ONE MED ROUTE

MENU TEXT: Map Local Medication Route to Standard

TYPE: run routine

DESCRIPTION:   This options provides the ability to select an entry from the

MEDICATION ROUTES (#51.2) File and map it to an entry in the STANDARD

MEDICATION ROUTES (#51.23) File.

ROUTINE: ONE^PSSMEDRT

UPPERCASE MENU TEXT: MAP LOCAL MEDICATION ROUTE TO

----------------------------------------------------------------

NAME: PSS MED ROUTE MAPPING REPORT

MENU TEXT: Medication Route Mapping Report

TYPE: run routine

DESCRIPTION:   This option provides a report that shows the status of the

current mapping from the Medication Routes (#51.2) File to the Standard

Medication Routes (#51.23) File.

ROUTINE: REP^PSSMEDRT

UPPERCASE MENU TEXT: MEDICATION ROUTE MAPPING REPOR

----------------------------------------------------------------

NAME: PSS MED ROUTE MAPPING CHANGES

MENU TEXT: Medication Route Mapping History Report

TYPE: run routine

DESCRIPTION:   This option will provide a report that shows all of the mapping

changes between entries from the MEDICATION ROUTES (#51.2) File to entries in

the STANDARD MEDICATION ROUTES (#51.23) File.

ROUTINE: DRIVER^PSSMEDCH

UPPERCASE MENU TEXT: MEDICATION ROUTE MAPPING HISTO

----------------------------------------------------------------

NAME: PSS MEDICATION ROUTE REQUEST

MENU TEXT: Request Change to Standard Medication Route

TYPE: run routine

DESCRIPTION:   This option enables a request to be made to have a new entry

added, or a current entry changed in the STANDARD MEDICATION ROUTES (#51.23)

File.

ROUTINE: REQ^PSSMEDRQ

UPPERCASE MENU TEXT: REQUEST CHANGE TO STANDARD MED

----------------------------------------------------------------

NAME: PSS LOCAL DOSAGES EDIT ALL

MENU TEXT: Find Unmapped Local Possible Dosages

TYPE: run routine

DESCRIPTION:   This option loops through and finds entries in the LOCAL

POSSIBLE DOSAGE (#50.0904) Subfile of the DRUG (#50) File without data

populated in the DOSE UNIT (#4) Field and the NUMERIC DOSE (#5) Field, and

will prompt for data entry into these fields. This data needs to be populated

if Dosage checks are to be performed, when that Local Possible Dosage is

selected for an order.

ROUTINE: EN^PSSLDALL

UPPERCASE MENU TEXT: FIND UNMAPPED LOCAL POSSIBLE D

----------------------------------------------------------------

NAME: PSS LOCAL DOSAGES EDIT

MENU TEXT: Map Local Possible Dosages

TYPE: run routine

DESCRIPTION:   This option allows edits to the DOSE UNIT (#4) Field and the

NUMERIC DOSE (#5) Field that are associated with entries from the LOCAL

POSSIBLE DOSAGE (#50.0904) Subfile of the DRUG (#50) File. This data needs to

be populated if Dosage checks are to be performed, when that Local Possible

Dosage is selected for an order.

ROUTINE: EDT^PSSLDEDT

UPPERCASE MENU TEXT: MAP LOCAL POSSIBLE DOSAGES

----------------------------------------------------------------

NAME: PSS LOCAL POSSIBLE DOSAGES

MENU TEXT: Local Possible Dosages Report

TYPE: run routine

DESCRIPTION:   This option provides a report that displays entries from the

LOCAL POSSIBLE DOSAGE (#50.0904) Subfile of the DRUG (#50) File with missing

data in DOSE UNIT (#4) Field and the NUMERIC DOSE (#5) Field. This data needs

to be populated if Dosage checks are to be performed, when that Local Possible

Dosage is selected for an order.

ROUTINE: EN^PSSLDOSE

UPPERCASE MENU TEXT: LOCAL POSSIBLE DOSAGES REPORT

----------------------------------------------------------------

NAME: PSS STRENGTH MISMATCH

MENU TEXT: Strength Mismatch Report

TYPE: run routine

DESCRIPTION:   This option provides a report that shows all entries from the

DRUG (#50) File that have data in the STRENGTH (#901) Field that does not

match the data in the STRENGTH (#2) Field from the match in the VA PRODUCT

(#50.68) File.

ROUTINE: EN^PSSTRENG

UPPERCASE MENU TEXT: STRENGTH MISMATCH REPORT

----------------------------------------------------------------

NAME: PSS DOSE UNIT REQUEST

MENU TEXT: Request Change to Dose Unit

TYPE: run routine

DESCRIPTION:   This option enables a request to be made to have a new entry

added, or a current entry changed in the DOSE UNITS (#51.24) File.

ROUTINE: DOSE^PSSMEDRQ

UPPERCASE MENU TEXT: REQUEST CHANGE TO DOSE UNIT

----------------------------------------------------------------

NAME: PSS MARK PREMIX SOLUTIONS

MENU TEXT: Mark PreMix Solutions

TYPE: run routine

DESCRIPTION:   This option will be used to mark entries from the IV SOLUTIONS

(#52.7) File as PreMixes, by allowing editing of the PREMIX (#18) Field.

ROUTINE: EDIT^PSSPRUTL

UPPERCASE MENU TEXT: MARK PREMIX SOLUTIONS

----------------------------------------------------------------

NAME: PSS IV SOLUTION REPORT

MENU TEXT: IV Solution Report

TYPE: run routine

DESCRIPTION:   This report will display all entries from the IV SOLUTIONS

(#52.7) File, or just those entries marked as PreMixes in the PREMIX (#18)

Field.

ROUTINE: REP^PSSPRMIX

UPPERCASE MENU TEXT: IV SOLUTION REPORT

----------------------------------------------------------------

NAME: PSS SCHEDULE REPORT

MENU TEXT: Administration Schedule File Report

TYPE: run routine

DESCRIPTION:   This option provides a report of entries from the

ADMINISTRATION SCHEDULE (#51.1) File that shows whether or not data has been

entered in the FREQUENCY (IN MINUTES) (#2) Field for the entries. To perform

Dosage checks on medication orders, a frequency must be derived from the

Schedule of the order.

ROUTINE: EN^PSSSCHRP

UPPERCASE MENU TEXT: ADMINISTRATION SCHEDULE FILE R

----------------------------------------------------------------

NAME: PSS MED INSTRUCTION REPORT

MENU TEXT: Medication Instruction File Report

TYPE: run routine

DESCRIPTION:   This option provides a report of entries from the MEDICATION

INSTRUCTION (#51) File that shows whether or not data has been entered in the

FREQUENCY (IN MINUTES) (#31) Field. To perform Dosage checks on medication

orders, a frequency must be derived from the Schedule of the order.

ROUTINE: EN^PSSMIRPT

UPPERCASE MENU TEXT: MEDICATION INSTRUCTION FILE RE

The following option was retrieved from VA FileMan and reflects the new option added to PDM following the installation of PSS*1*140.

----------------------------------------------------------------

NAME: PSS MEDICATION ROUTES MGMT

MENU TEXT: Medication Routes Management

TYPE: menu

DESCRIPTION:  This Sub-Menu contains options related to Medication Routes in

both the MEDICATION ROUTES (#51.2) File and the STANDARD MEDICATION ROUTES

(#51.23) File.

ITEM: PSS MEDICATION ROUTES EDIT

ITEM: PSS MED ROUTE MAPPING REPORT

ITEM: PSS MED ROUTE MAPPING CHANGES

ITEM: PSS MEDICATION ROUTE REQUEST

This option introduced with PSS*1*140.

ITEM: PSS DEF MED ROUTE OI RPT

----------------------------------------------------------------

NAME: PSS DEF MED ROUTE OI RPT

MENU TEXT: Default Med Route For OI Report

TYPE: print

DIC {DIP}: PS(50.7,                   L.: 0

FLDS: [PSS DEF MED ROUTE FOR OI]      BY: [PSS DEF MED ROUTE FOR OI]

DHD: [PSS HDR DEF MED ROUTE]

UPPERCASE MENU TEXT: DEFAULT MED ROUTE FOR OI REPOR

----------------------------------------------------------------

The following options were retrieved from VA FileMan and reflect the new options added to PDM following the installation of patch PSS*1*147.

NAME: PSS IV ADDITIVE REPORT

MENU TEXT: IV Additive Report

TYPE: run routine

DESCRIPTION:   This report will display entries from the IV ADDITIVES (#52.6)

File. The display selection choices will be for all IV Additives, only IV

Additives whose ADDITIVE FREQUENCY (#18) Field is set to '1 BAG/DAY', or only

those IV Additives whose ADDITIVE FREQUENCY (#18) Field is null.

ROUTINE: REP^PSSADRPT

UPPERCASE MENU TEXT: IV ADDITIVE REPORT

----------------------------------------------------------------

NAME: PSS ADDITIVE/SOLUTION REPORTS

MENU TEXT: IV Additive/Solution Reports

TYPE: menu

DESCRIPTION:   This Sub-Menu contains options that can be used to run reports

from the IV ADDITIVES (#52.6) File and the IV SOLUTIONS (#52.7) File.

ITEM: PSS IV ADDITIVE REPORT            DISPLAY ORDER: 1

ITEM: PSS IV SOLUTION REPORT            DISPLAY ORDER: 2

UPPERCASE MENU TEXT: IV ADDITIVE/SOLUTION REPORTS

## Routines

The following routines are used by the Pharmacy Data Management package.

| PSS129EN | PSS147EN | PSS147PO | PSS32P3 | PSS32P5 | PSS50 |
| --- | --- | --- | --- | --- | --- |
| PSS50A | PSS50A1 | PSS50AQM | PSS50ATC | PSS50B | PSS50B1 |
| PSS50B2 | PSS50C | PSS50C1 | PSS50CMP | PSS50D | PSS50DAT |
| PSS50DOS | PSS50E | PSS50F | PSS50F1 | PSS50LAB | PSS50NDF |
| PSS50P4 | PSS50P66 | PSS50P7 | PSS50P7A | PSS50TMP | PSS50WS |
| PSS51 | PSS51P1 | PSS51P15 | PSS51P1A | PSS51P1B | PSS51P1C |
| PSS51P2 | PSS51P5 | PSS52P6 | PSS52P6A | PSS52P6B | PSS52P7 |
| PSS52P7A | PSS54 | PSS55 | PSS551 | PSS55MIS | PSS59P7 |
| PSS781 | PSSADDIT | PSSADRPT | PSSAUTL | PSSCHENV | PSSCHPRE |
| PSSCHPST | PSSCLDRG | PSSCOMMN | PSSCPRS | PSSCPRS1 | PSSCREAT |
| PSSCSPD | PSSDAWUT | PSSDDUT | PSSDDUT2 | PSSDDUT3 | PSSDEE |
| PSSDEE1 | PSSDEE2 | PSSDELOI | PSSDENT | PSSDFEE | PSSDGUPD |
| PSSDI | PSSDIN | PSSDOS | PSSDOSCR | PSSDOSCX | PSSDOSED |
| PSSDOSER | PSSDOSRP | PSSDSAPA | PSSDSDAT | PSSDSPON | PSSDSPOP |
| PSSDTR | PSSENV | PSSENVN | PSSFIL | PSSFILED | PSSFILES |
| PSSGENM | PSSGIU | PSSGMI | PSSGS0 | PSSGSH | PSSHELP |
| PSSHL1 | PSSHLSCH | PSSHLU | PSSJEEU | PSSJORDF | PSSJSPU |
| PSSJSPU0 | PSSJSV | PSSJSV0 | PSSJXR | PSSJXR1 | PSSJXR10 |
| PSSJXR11 | PSSJXR12 | PSSJXR13 | PSSJXR14 | PSSJXR15 | PSSJXR16 |
| PSSJXR17 | PSSJXR18 | PSSJXR19 | PSSJXR2 | PSSJXR20 | PSSJXR21 |
| PSSJXR22 | PSSJXR4 | PSSJXR5 | PSSJXR6 | PSSJXR7 | PSSJXR8 |
| PSSJXR9 | PSSLAB | PSSLDALL | PSSLDEDT | PSSLDOSE | PSSLDOSE |
| PSSLOOK | PSSMARK | PSSMATCH | PSSMEDCH | PSSMEDRQ | PSSMEDRT |
| PSSMEDX | PSSMIRPT | PSSMRTUP | PSSMRTUX | PSSMSTR | PSSNDCUT |
| PSSNOUNR | PSSNTEG | PSSOICT | PSSOICT1 | PSSOPKI | PSSOPKI1 |
| PSSORPH | PSSORPH1 | PSSORUTL | PSSOUTSC | PSSPKIPI | PSSPKIPR |
| PSSPNSRP | PSSPO129 | PSSPOI | PSSPOIC | PSSPOID1 | PSSPOID2 |
| PSSPOID3 | PSSPOIDT | PSSPOIKA | PSSPOIM | PSSPOIM1 | PSSPOIM2 |
| PSSPOIM3 | PSSPOIMN | PSSPOIMO | PSSPOST | PSSPOST1 | PSSPOST2 |
| PSSPOST5 | PSSPRE | PSSPRETR | PSSPRMIX | PSSPRUTL | PSSQORD |
| PSSREF | PSSREMCH | PSSSCHED | PSSSCHRP | PSSSOLI1 | PSSSOLIT |
| PSSSPD | PSSSUTIL | PSSSYN | PSSTRENG | PSSUTIL | PSSUTLA1 |
| PSSUTLPR | PSSVIDRG | PSSVX6 | PSSVX61 | PSSVX62 | PSSVX63 |
| PSSVX64 | PSSVX65 | PSSVX66 | PSSWMAP | PSSWRNA | PSSWRNB |
| PSSWRNE | PSSYSP |  |  |  |  |

(This page included for two-sided copying.)

## Exported Options

#### Stand Alone Options

All of the PDM options are designed to stand-alone and can be accessed without first accessing the top-level menu.

#### Menus

The original PDM menu that would be seen following the initial setup of the PDM package is displayed below. The options needed to build and maintain the PHARMACY ORDERABLE ITEM file (#50.7) will be under the Pharmacy Data Management menu, and will appear as follows.

Pharmacy Data Management [PSS MGR]

Primary/VA Generic Orderable Item Report

[PSS PRIMARY/VA GENERIC REPORT]

VA Generic Orderable Item Report

[PSS VA GENERIC DRUG REPORT]

Create Pharmacy Orderable Items

[PSS CREATE ORDERABLE ITEMS]

Manually Match Dispense Drugs

[PSS PRE-RELEASE MANUAL MATCH]

Orderable Item Matching Status

[PSS ORDERABLE ITEM STATUS]

CMOP Mark/Unmark (Single drug)

[PSSXX MARK]

Drug Enter/Edit

[PSS DRUG ENTER/EDIT]

Drug Interaction Management...

[PSS DRG INTER MANAGEMENT]

Enter/Edit Local Drug Interaction

[PSS INTERACTION LOCAL ADD]

Edit Drug Interaction Severity

[PSS INTERACTION SEVERITY]

Electrolyte File (IV)

[PSSJI ELECTROLYTE FILE]

Lookup into Dispense Drug File

[PSS LOOK]

Med. Route/Instructions Table Maintenance

[PSS MED. ROUTE/INSTRUCTIONS]

Medication Instruction File Add/Edit

[PSSJU MI]

Orderable Item Management...

[PSS ORDERABLE ITEM MANAGEMENT]

Edit Orderable Items

[PSS EDIT ORDERABLE ITEMS]

Dispense Drug/Orderable Item Maintenance

[PSS MAINTAIN ORDERABLE ITEMS]

Additive/Solutions, Orderable Items

[PSS ADDITIVE/SOLUTIONS]

Orderable Item Report

[PSS ORDERABLE ITEM REPORT]

Primary Drug Edit

[PSS PRIMARY DRUG EDIT]

Pharmacy System Parameters Edit

[PSS SYS EDIT]

Standard Schedule Edit

[PSS SCHEDULE EDIT]

Locked: PSXCMOPMGR

Without the PSXCMOPMGR key, the CMOP Mark/Unmark (Single drug) option will not appear on your menu.

The PDM menu that was exported with the original PDM package has been modified to include subsequent changes and patches.

The PDM menu up to and including PSS*1*147 appears below. PSS*1*147 was the last patch to affect a change to the PDM menu.

When using the CMOP Mark/Unmark (Single drug) [PSSXX MARK] option to mark a drug for CMOP the user must update the cost information in the DRUG file (#50) to ensure the cost data for each prescription fill will be correct.

Pharmacy Data Management [PSS MGR]

CMOP Mark/Unmark (Single drug)

[PSSXX MARK]

Dosages…

[PSS DOSAGES MANAGEMENT]

Auto Create Dosages

[PSS DOSAGE CONVERSION]

Dosage Form File Enter/Edit

[PSS DOSAGE FORM EDIT]

Enter/Edit Dosages

[PSS EDIT DOSAGES]

Most Common Dosages Report

[PSS COMMON DOSAGES]

Noun/Dosage Form Report

[PSS DOSE FORM/ NOUN REPORT]

Review Dosages Report

[PSS DOSAGE REVIEW REPORT]

Local Possible Dosages Report

[PSS LOCAL POSSIBLE DOSAGES]

Request Change to Dose Unit

[PSS DOSE UNIT REQUEST]

Drug Enter/Edit

[PSS DRUG ENTER/ EDIT]

Drug Interaction Management…

[PSS DRG INTER MANAGEMENT]

Enter/Edit Local Drug Interaction

[PSS INTERACTION LOCAL ADD]

Report of Locally Entered Interactions

[PSS REPORT LOCAL INTERACTIONS]

Electrolyte File (IV)

[PSSJI ELECTROLYTE FILE]

Lookup into Dispense Drug File

[PSS LOOK]

Medication Instruction Management ...

[PSS MED INSTRUCTION MANAGEMENT]

Medication Instruction File Add/Edit  <<Moved from main PSS MGR menu

[PSSJU MI]

Medication Instruction File Report

[PSS MED INSTRUCTION REPORT]

Medication Routes Management ...

[PSS MEDICATION ROUTES MGMT]

Medication Route File Enter/Edit  <<Moved from main PSS MGR menu

[PSS MEDICATION ROUTES EDIT]

Medication Route Mapping Report

[PSS MED ROUTE MAPPING REPORT]

Medication Route Mapping History Report

[PSS MED ROUTE MAPPING CHANGES]

Request Change to Standard Medication Route

[PSS MEDICATION ROUTE REQUEST]

Default Med Route for OI Report

[PSS DEF MED ROUTE OI RPT]

Orderable Item Management

[PSS ORDERABLE ITEM MANAGEMENT]

Edit Orderable Items

[PSS EDIT ORDERABLE ITEMS]

Dispense Drug/Orderable Item Maintenance

[PSS MAINTAIN ORDERABLE ITEMS]

Orderable Item/Dosages Report

[PSS ORDERABLE ITEM DOSAGES

Patient Instructions Report

[PSS INSTRUCTIONS/ ITEMS REPORT]

Orderable Item Report <<Moved from main PSS MGR menu

[PSS ORDERABLE ITEM REPORT]

Formulary Information Report

[PSSNFI]

Drug Text Management ...

[PSS DRUG TEXT MANAGEMENT]

Drug Text Enter/Edit  <<Moved from main PSS MGR menu

[PSS EDIT TEXT]

Drug Text File Report  <<Moved from main PSS MGR  menu

[PSS DRUG TEXT FILE REPORT

Pharmacy System Parameters Edit

[PSS SYS EDIT]

Standard Schedule Management ...

[PSS SCHEDULE MANAGEMENT]

Standard Schedule Edit  <<Moved from main PSS MGR menu

[PSS SCHEDULE EDIT]

Administration Schedule File Report

[PSS SCHEDULE REPORT]

Synonym Enter/Edit

[PSS SYNONYM EDIT]

The Other Language Translation Setup [PSS OTHER LANGUAGE SETUP] option is a stand-alone option that must be assigned to the person(s) responsible for maintaining it.

Other Language Translation Setup

[PSS OTHER LANGUAGE SETUP]

Controlled Substances/PKI Reports

[PSS CS/PKI REPORTS]

DEA Spec Hdlg & CS Fed Sch Discrepancy

[PSS DEA VS CS FED. SCH. DISCR.]

Controlled Substances Not Matched to NDF

[PSS CS NOT MATCHED TO NDF]

CS (DRUGS) Inconsistent with DEA Spec Hdlg

[PSS CS DRUGS INCON WITH DEA]

CS (Ord. Item) Inconsistent with DEA Spec Hdlg

[PSS CS (OI) INCON WITH DEA]

Send Entire Drug File to External Interface

[PSS MASTER FILE ALL]

Enhanced Order Checks Setup Menu ...

[PSS ENHANCED ORDER CHECKS]

Find Unmapped Local Medication Routes

[PSS MED ROUTES INITIAL MAPPING]

Map Local Medication Route to Standard

[PSS MAP ONE MED ROUTE]

Medication Route Mapping Report

[PSS MED ROUTE MAPPING REPORT]

Medication Route File Enter/Edit

[PSS MEDICATION ROUTES EDIT]

Medication Route Mapping History Report

[PSS MED ROUTE MAPPING CHANGES]

Request Change to Standard Medication Route

[PSS MEDICATION ROUTE REQUEST]

Find Unmapped Local Possible Dosages

[PSS LOCAL DOSAGES EDIT ALL]

Map Local Possible Dosages

[PSS LOCAL DOSAGES EDIT]

Local Possible Dosages Report

[PSS LOCAL POSSIBLE DOSAGES]

Strength Mismatch Report

[PSS STRENGTH MISMATCH]

Enter/Edit Dosages

[PSS EDIT DOSAGES]

Request Change to Dose Unit

[PSS DOSE UNIT REQUEST]

Mark PreMix Solutions

[PSS MARK PREMIX SOLUTIONS]

IV Solution Report

[PSS IV SOLUTION REPORT]

Administration Schedule File Report

[PSS SCHEDULE REPORT]

Medication Instruction File Report

[PSS MED INSTRUCTION REPORT]

IV Additive/Solution Reports…

[PSS ADDITIVE/SOLUTION REPORTS]

IV Additive Report

[PSS IV ADDITIVE REPORT]

IV Solution Report

[PSS IV SOLUTION REPORT]

Warning Builder

[PSS WARNING BUILDER]

Warning Mapping

[PSS WARNING MAPPING]

Locked: PSXCMOPMGR

Without the PSXCMOPMGR key, the CMOP Mark/Unmark (Single drug) option will not appear on your menu.

A post-installation routine for patch PSS*1*61 identifies those drugs with discrepancies between the CS FEDERAL SCHEDULE field (#19) of the VA PRODUCT file (#50.68) and DEA, SPECIAL HDLG field (#3) of the DRUG file (#50). Four mail messages are generated and sent to the installer and to holders of the PSNMGR keys: These mail messages can be re-generated on demand using the Controlled Substances/PKI Reports [PSS CS/PKI REPORTS] menu added with this patch.

The post-install also checks to see if the DEA, SPECIAL HDLG field (#3) of the DRUG file (#50) is null and if the drug is matched to an entry in the VA PRODUCT file (#50.68), and the CS FEDERAL SCHEDULE field (#19) of the VA PRODUCT file (#50.68) has data, then it updates the DEA, SPECIAL HDLG field (#3) of the DRUG file (#50) with the DEA, SPECIAL HDLG code mapped to corresponding CS FEDERAL SCHEDULE code.

## Protocols

NAME: PSS EXT MFU CLIENT

DESCRIPTION:   This protocol will be used as the ACK from the external

interface for a MFN_M01 message.

NAME: PSS EXT MFU SERVER

DESCRIPTION:   This protocol will be used to send even notification and data

when new drugs are added to the DRUG file (#50) and when certain fields are

updated in the same file.  This information will be sent to the automated

dispensing machines through HL7 V.2.4 formatted messages.

NAME: PSS HUI DRUG UPDATE

DESCRIPTION:   This protocol will be used to send event notification and data

when new drugs are added to the Drug file (#50) and when certain fields are

updated in same file.

NAME: PSS MED ROUTE RECEIVE

DESCRIPTION:   This protocol processes updates to the Standard Medication

Routes (#51.23) File.

## HL7 Messaging with an External System

#### New Protocol

A new protocol, PSS HUI DRUG UPDATE, is exported and has been created to generate HL7 messages when new drugs are added to the DRUG file (#50) and existing entries are updated. This protocol is exported with the text “DELETE ONLY TO SEND DRUG UPDATE MESSAGES” in the DISABLE field (#2) of the PROTOCOL file (#101). To activate the sending of these HL7 messages, the text from the DISABLE field (#2) of the PROTOCOL file (#101) must be deleted and at least one receiving protocol added as a subscriber. The drug data elements included in the HL7 message are defined in the table below.

#### HL7 Drug Message Segment Definition Table

When the PSS HUI DRUG UPDATE protocol is enabled, the following table defines the data elements sent in each segment of the HL7 drug message.

| Segment | Piece | Field Name | HL7 TBL # or Data Type | Description |
| --- | --- | --- | --- | --- |
| MSH | 1 | ¦ | ST | Field Separator |
|  | 2 | ^~\& | ST | Encoding Characters |
|  | 3 | Pharmacy | No suggested value | Sending Application |
|  | 9 | MFN | 0076 | Message Type |
| MFI | 1 | 50^DRUG^99PSD | 0175 | Master File ID |
|  | 3 | UPD | 0178 | File-Level Event Code |
|  | 6 | NE | 0179 | Response Level Code |
| MFA | 1 | MUP/MAD | 0180 | UPDATE/ADD |
| MFE | 1 | MUP/MAD | 0180 | UPDATE/ADD |
|  | 4 | IEN^DRUG NAME^99PSD |  | File 50 Entry |
| ZPA | 1 | NDC | ST | National Drug Code |
|  | 2 | LOCAL NON-FORMULARY | CE | If “1” true |
|  | 3 | INACTIVE DATE | DT | HL7 Format (YYYYMMDD) |
|  | 4 | APPLICATION PACKAGE USE | ST | Used by what packages |
|  | 5 | MESSAGE | ST | Info on drug |
|  | 6 | VA CLASSIFICATION | ST | VA Class |
|  | 7 | DEA SPECIAL HDLG | ST | How drug is dispense based on DEA codes |
|  | 8 | FSN | ST | Federal Stock # |
|  | 9 | WARNING LABEL | ST | Drug Warnings for patient |
|  | 10 | VISN NON-FORMULAR | CE | If ‘1’ true |

| Segment | Piece | Field Name | HL7 TBL # or Data Type | Description |
| --- | --- | --- | --- | --- |
| ZPB | 1 | PHARMACY ORDERABLE ITEM | CE | IEN^OI tied to dispense drug^PSD50.7 |
|  | 2 | DOSAGE FORM | ST | IEN^Dosage Form associated with OI^PSD50.606 |
|  | 3 | MEDICATION ROUTE | ST | IEN^Med Route associated with OI^PSD51.2 |
|  | 4 | PSNDF VA PRODUCT NAME ENTRY | CE | IEN^VA PRODUCT NAMES^PSD50.68 |
|  | 5 | DISPENSE UNIT | ST | Dispense Unit for a drug |
|  | 6 | CMOP DISPENSE | CE | 1 or 0 |
|  | 7 | OP EXTERNAL DISPENSE | CE | 1 or 0 |
|  | 8 | EXPIRATION DATE | DT | HL7 Format (YYYYMMDD) |
|  | 9 | LAB TEST MONITOR | CE | IEN^Lab Test^LAB60 |
| ZPC | 1 | SPECIMEN TYPE | CE | IEN^ SPECIMEN TYPE^LAB61 |
|  | 2 | MONITOR ROUTINE | ST | Program that runs to find lab test and results |
|  | 3 | LAB MONITOR MARK | CE | If ‘1’ true |
|  | 4 | STRENGTH | NM | Dose of drug |
|  | 5 | UNIT | CE | IEN^Unit of measure^PSD50.607 |
|  | 6 | PRICE PER ORDER UNIT | NM |  |
|  | 7 | PRICE PER DISPENSE UNIT | NM |  |
| [{ZPD}] | 1 | SYNONYM | ST | Trade Name |
|  | 2 | NDC CODE | ST | National Drug Code |
|  | 3 | INTENDED USE | CE | CE^INTENTED USE |
|  | 4 | VSN | ST | Vendor Stock Number |
|  | 5 | ORDER UNIT | CE | IEN^ABBREVIATION^EXPANSION ^PSD51.5 |
|  | 6 | PRICE PER ORDER UNIT | NM |  |
|  | 7 | DISPENSE UNITS PER ORDER UNIT | NM |  |
|  | 8 | PRICE PER DISPENSE UNIT | NM |  |
|  | 9 | VENDOR | ST | Vendor |
| [{ZPE}] | 1 | ACTIVITY LOG | DT | HL7 Format YYYYMMDDHHMM[SS]-ZZZZ |
|  | 2 | REASON | CE | E^EDIT |
|  | 3 | INITIATOR OF ACTIVITY | CE | IEN^NEW PERSON^VA200 |
|  | 4 | FIELD EDITED | ST |  |
|  | 5 | NEW VALUE | ST |  |
|  | 6 | NDF UPDATE | ST |  |
| [{ZPF}] | 1 | DISPENSE UNITS PER DOSE | NM |  |
|  | 2 | DOSE | NM |  |
|  | 3 | PACKAGE | CE | CE^PACKAGE(S) |
|  | 4 | BCMA UNITS PER DOSE | NM |  |
| [{ZPG}] | 1 | CLOZAPINE LAB TEST | CE | IEN^LAB TEST^LAB60 |
|  | 2 | MONITOR MAX DAYS | NM |  |
|  | 3 | SPECIMEN TYPE | CE | IEN^ SPECIMEN TYPE^LAB61 |
|  | 4 | TYPE OF TEST | CE | 1^WBC or 2^ANC |
| [{ZPH}] | 1 | LOCAL POSSIBLE DOSAGE | ST | FREE TEXT |
|  | 2 | PACKAGE | CE | CE^PACKAGE(S) |
|  | 3 | BCMA UNITS PER DOSE | NM |  |

#### New Protocol

Two new protocols, PSS EXT MFU CLIENT and PSS EXT MFU SERVER, are exported and have been created to generate HL7 messages when new drugs are added to the DRUG file (#50) and existing entries are updated. These protocols can only be activated by setting the following parameters in the OUTPATIENT SITE file (#59):

- AUTOMATED DISPENSE field (#105) needs to be set to 2.4.

- ENABLE MASTER FILE UPDATE field (#105.2) needs to be set to YES.

- LOGICAL LINK field (#2005) needs to be set to PSO DISP.

- DISPENSE DNS NAME field (#2006) needs to be set to the dispensing system DNS name (for example, dispensemachine1.vha.med.va.gov).

- DISPENSE DNS PORT field (#2007) needs to be set to the dispensing system port number.

Specific Transaction

The Pharmacy/Treatment Encoded Order Message is as follows:

MFN	Master File Notification Message

MSH	Message Header

MFI	Master File Identifier

{MFE	Master File Entry

{{ZPA}	Drug File Information

{RXD}	Pharmacy/Treatment Dispense

{OBR}}		Observation Request

}

Example:

MSH|~^\&|PSS VISTA|521~FO-BIRM.VHA.MED.VA.GOV~DNS|PSS DISPENSE|~DISPENSE1.VHA.MED.VA.GOV:9300~DNS|20030701||MFN~M01~MFN_M01|10001|P|2.4|||AL|AL

MFI|50~DRUG~99PSD||UPD|||NE

MFE|MUP|||PROPANTHELINE 15MG TAB

ZPA|PROPANTHELINE 15MG TAB|N|LFN~Local Non-Formulary~Pharm Formulary Listing|20031226|Take with food|DE200|6|P|50~6505-00-960-8383~LPS50|8~NO ALCOHOL~LPS54|229~Bacitracin~LPSD50.7|3~CAP,ORAL~LPSD50.606|15~IV PUSH~LPSD51.2|3643~ATROPINE SO4 0.4MG TAB~LPSD50.68|OP~OP Dispense~99OP|20030830|9~Rubella~LLAB60|72~Hair of Scalp~LLAB61|PSOCLO1|N|100|20~MG~LPSD50.607|4.28&USD~UP|15.64&USD~UP|TAB|2|BLUE HOUSE VENDOR|0010-0501-33|TRADENAME

RXD||||1|||||1|||~P&200&LPSD50.0903||||||||||||O

OBR||||1102~ACETAZOLAMIDE~LLAB60|||||||||||70&NECK&LLAB61|||||||||WBC|||7

#### HL7 Drug Message Segment Definition Table

When the PSS EXT MFU SERVER protocol is enabled, the following table defines the data elements sent in each segment of the HL7 drug message.

Segments used in the Master File Update message:

| SEGMENT | SEQ# | LEN | DT | R/O | RP/# | TBL# | ELEMENT NAME | EXAMPLE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MSH | 1 | 1 | ST | R |  |  | Field Separator | ¦ |
|  | 2 | 4 | ST | R |  |  | Encoding Characters | ~^\& |
|  | 3 | 180 | HD | R |  | 0361 | Sending Application | PSS VISTA |
|  | 4 | 180 | HD | R |  | 0362 | Sending Facility – station ID and station DNS name | 521~FO-BIRM.MED.VA.GOV~DNS |
|  | 5 | 180 | HD | R |  | 0361 | Receiving Application | PSS DISPENSE |
|  | 6 | 180 | HD | R |  | 0362 | Receiving Facility – DNS name and port of dispensing machine | ~DISPENSE.VHA.MED.VA.GOV:9300~DNS |
|  | 7 | 26 | TS |  |  |  | Date/Time of Message | 20040405152416 |
|  | 9 | 15 | CM | R | 0076 |  | Message Type | MFN_M01 |
|  | 10 | 20 | ST | R |  |  | Message Control ID | 10001 |
|  | 11 | 3 | PT | R | 0103 |  | Processing ID | P |
|  | 12 | 3 | VID | R | 0104 |  | Version ID | 2.4 |
|  | 15 | 2 | ID |  |  | 0155 | Accept Ack. Type | AL |
|  | 16 | 2 | ID |  |  | 0155 | Application Ack Type | AL |
| MFI | 1 | 250 | CE | R |  | 0175 | Master File Identifier | 50^DRUG^99PSD |
|  | 3 | 3 | ID | R |  | 0178 | File-Level Event Code | UPD |
|  | 6 | 2 | ID | R |  | 0179 | Response Level Code | NE |
| MFE | 1 | 3 | ID | R |  | 0180 | Record-Level Event Code | MUP |
|  | 4 | 200 | Varies | R |  |  | Primary Key Value – MFE | PROPANTHELINE 15MG TAB |
| ZPA | 1 | 200 | Varies | R |  |  | Primary Key Value – ZPA | PROPANTHELINE 15MG TAB |
|  | 2 | 1 | ID | R |  | 0136 | Is Synonym | N |
|  | 3 | 200 | CE | R |  |  | Formulary Listing | LFN~Local Non-Formulary~Pharm Formulary Listing |
|  | 4 | 10 | DT | O |  |  | Inactive Date | 20031226 |
|  | 5 | 200 | ST | O |  |  | Drug Message | Take with Food |
|  | 6 | 30 | ST | O |  |  | Drug Classification | DE200 |
|  | 7 | 10 | ST | O |  |  | DEA-Schedule Code | 6 |
|  | 8 | 1 | ST | O |  |  | DEA-Drug Type | P |
|  | 9 | 100 | CE | R |  |  | Stock Number | 50~6505-00-960-8383~LPS50 |
|  | 10 | 100 | CE | O |  |  | Warning Label | 8~NO ALCOHOL~LPS54 |

Segments used in the Master File Update message: (continued)

| SEGMENT | SEQ# | LEN | DT | R/O | RP/# | TBL# | ELEMENT NAME | EXAMPLE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11 | 100 | CE | O |  |  | Pharmacy Orderable Item | 229~Bacitracin~LPSD50.7 |
|  | 12 | 100 | CE | O |  |  | Dosage Form | 3~CAP,ORAL~LPSD50.606 |
|  | 13 | 100 | CE | O |  |  | Medication Route | 15~IV PUSH~LPSD51.2 |
|  | 14 | 100 | CE | O |  |  | Drug Name Identifiers | 3643~ATROPINE SO4 0.4MG TAB~LPSD50.68 |
|  | 15 | 100 | CE | O |  |  | Dispense Flags | OP~OP Dispense~99OP |
|  | 16 | 15 | DT | O |  |  | Drug Expiration Date | 20030830 |
|  | 17 | 100 | CE | O |  |  | Lab Test Monitor | 9~Rubella~LLAB60 |
|  | 18 | 100 | CE | O |  |  | Specimen Type | 72~Hair of Scalp~LLAB61 |
|  | 19 | 10 | CE | O |  |  | Monitor Routine | PSOCLO1 |
|  | 20 | 1 | ID | O |  |  | Lab Monitor Mark | N |
|  | 21 | 50 | NM | O |  |  | Strength | 100 |
|  | 22 | 250 | CE | R |  |  | Unit | 20~MG~LPSD50.607 |
|  | 23 | 50 | CP | R |  |  | Price Per Order Unit | 4.28&USD~UP |
|  | 24 | 50 | CP | R |  |  | Price Per Dispense Unit | 15.64&USD~UP |
|  | 25 | 25 | ST | O |  |  | Dispense Unit | TAB |
|  | 26 | 50 | NM | O |  |  | Dispense Units Per Order Unit | 2 |
|  | 27 | 50 | ST | O |  |  | Vendor | BLUE HOUSE VENDOR |
|  | 28 | 12 | ST | O |  |  | NDC Code | 0010-0501-33 |
|  | 29 | 25 | ST | O |  |  | Intended Use | TRADE NAME |
| RXD | 4 | 20 | NM | R |  |  | Actual Dispense Amount | 1 |
|  | 8 | 20 | NM | R |  |  | Dispense Notes | 1 |
|  | 12 | 10 | CQ | O |  |  | Total Daily Dose | ~P&200&LPSD50.0903 |
|  | 24 | 2 | ID | R |  |  | Dispense Package Method | O |
| OBR | 4 | 250 | CE | O |  |  | Universal Service Identifier | 1102~ACETAZOLAMIDE~LLAB60 |
|  | 15 | 300 | CM | O |  |  | Specimen Source | 70&NECK&LLAB61 |
|  | 24 | 3 | ID | R |  |  | Diagnostic Serv Sect ID | WBC |
|  | 27 | 200 | TQ | O |  |  | Quantity/Timing | 7 |

Notes pertaining to some of the data elements:

[MSH-3] Sending Application is the station ID along with the DNS name of the sending facility.

[MSH-5] Receiving Application is the DNS name and DNS port number of the dispensing application.

[MSH-10] Message Control ID is the number that uniquely identifies the message. It is returned in MSA-2 of the dispense completion message.

[MFI-1] Master File Identifier is hard-coded to 50~DRUG~99PSD.

[MFE-1] Record-Level Event Code can be either MUP for Update or MAD for Add.

[MFE-4] Primary Key Value – MFE is the GENERIC NAME field (#.01) from the DRUG file (#50).

[ZPA-1] Primary Key Value – ZPA will be the generic name of the drug first and then all synonyms will follow in consecutive ZPA segments.

[ZPA-2] Is Synonym is set to Y or N depending on whether the primary key is a synonym.

[ZPA-3] Formulary Listing will contain LFN and/or VISN is the formulary is not to appear on the Local or VISN formulary.

[ZPA-9] Stock Number is the FSN field (#6) from the DRUG file (#50) or the VSN field (#400) from the SYNONYM subfile (#50.1) of the PRESCRIPTION file (#50).

[ZPA-15] Dispense Flags will indicate if this drug may be dispensed to an external interface and if it is marked to be dispensed at a Consolidated Outpatient Pharmacy (CMOP). If both are yes, the answer would be OP~OP Dispense~Pharm dispense^CMOP~CMOP dispense~Pharm dispense flag.

[ZPA-29] Intended User will be TRADE NAME, QUICK CODE, DRUG ACCOUNTABILITY or CONTROLLED SUBSTANCES.

[RXD-4] Actual Dispense Amount is the BCMA UNITS PER DOSE field (#3) from the POSSIBLE DOSAGES file (#50.0903).

[RXD-9] Dispense Notes is the DISPENSE UNITS PER DOSE field (#.01) from the POSSIBLE DOSAGES file (#50.0903).

[RXD-12] Total Daily Dose will be either P for Possible Dosages or LP for Local Possible Dosages.

[OBR-4] Universal Service Identifier is used for Clozapine Lab Test.

[OBR-15] Specimen Source is used for Clozapine Specimen Type.

[OBR-24] Diagnostic Serv Sect ID is used for Clozapine Type of Test.

[OBR-27] Quantity/Timing is used to encode Monitor Max days from the CLOZAPINE LAB TEST file (#50.02).

## Data Archiving and Purging

There are no archiving and purging functions necessary with this release of the PDM package.

## Callable Routines/Entry Points/Application Program Interfaces (APIs)

APIs, callable routines, and entry points can be viewed by first choosing the DBA menu option on FORUM and then choosing the Integration Agreements Menu option.

For detailed information on all supported Pharmacy Data Management APIs, see the Pharmacy Re-Engineering (PRE) Application Program Interface (API) Manual posted on the VistA Documentation Library (VDL) at http://www.va.gov/vdl/.

## Medication Routes

The following paragraphs provide an explanation of medication route information.

If there is one and only one orderable item in the IV order, the Pharmacy Orderable Item file (50.7) will be checked to see if there is a default med route. Additionally, any med route defined for the dosage form of the orderable item will be checked. Duplicate entries will be filtered by inpatient pharmacy. The list of possible med routes sent back to CPRS will include the default med route (if defined), as well as any other med routes defined for the dosage form.

If there is more than one orderable item on the IV order, the Pharmacy Orderable Item file will be checked for each orderable item to see if a default is defined. This default will be pooled with the other possible med routes for each dosage form for each orderable item. Duplicates will be filtered by inpatient pharmacy. If all of the orderable items on the order have the same default med route defined, the default med route will be denoted in the selection list in CPRS. Regardless of whether all orderable item defaults match or not, a default med route will not be marked as a default if there is more than one orderable item on the order. The list of possible med routes returned to CPRS will include only those med routes that every orderable item in the order shares in common.

## Administration Scheduling

The following rules apply to administration scheduling.

If there is a duplicate schedule, and if one of them contains ward-specific administration times for the ward location of the patient, the schedule returned for inclusion in the array of selectable schedules in CPRS will be the one with the ward-specific administration times.

If no duplicate has ward-specific administration times for the ward location of the patient, the schedule with the lowest IEN number will be returned. If both (or more than one) duplicate schedules have ward-specific administration times for the ward location of the patient, the schedule with the lowest IEN number in the ADMINISTRATION SCHEDULE file #51.1 will be the schedule in the array returned to CPRS.

## External Relations

#### Integration Agreements

IAs can be viewed by first choosing the DBA option on FORUM and then the Integration Agreements Menu option.

Example: DBA Option

Select Primary Menu Option: DBA

Select DBA Option: INTEGration Agreements Menu

Select Integration Agreements Menu Option:  Custodial Package Menu

Select Custodial Package Menu Option:  ACTIVE by Custodial Package

Select PACKAGE NAME: PHARMACY DATA MANAGEMENT     PSS

DEVICE: HOME//

## Internal Relations

All PDM options can function independently.

## Package-Wide Variables

There are no package-wide variables for this version.

## Package Requirements

The initial PDM module relies on, at least, the following external packages to run effectively.

Package	Minimum version needed

National Drug File	V. 3.15

Outpatient Pharmacy	V. 6.0

Inpatient Medications	V. 4.5

Kernel	V. 8.0 (plus all patches, particularly XU*8*28)

VA FileMan		V. 21.0 (plus all patches)

The PDM module up to and including the release of PSS*1*68, relies on, at least, the following external packages to run effectively.

Package	Minimum version needed

National Drug File	V. 4.0

Outpatient Pharmacy	V. 7.0

Inpatient Medications	V. 5.0

Kernel	V. 8.0

VA FileMan		V. 22

(This page included for two-sided copying.)

## Additional Information

#### SAC Exemptions

There are no Standards and Conventions (SAC) exemptions for this package.

#### Cross-Reference Logic To Keep Orderable Items Up To Date

With the introduction of PSS*1*38, a new process for keeping Orderable Items updated was implemented. The process is explained in detail in the section below.

Anytime specific fields are edited, or a pointer to the PHARMACY ORDERABLE ITEM file (#50.7) changes, the Orderable Item (OI) must be updated and sent to CPRS. Two different situations can precipitate these changes. Both situations are explained in detail here.

The first situation occurs when a field is edited that can possibly affect the status of the Orderable Item, but no Orderable Item pointers change. In this situation, the old Orderable Item is the same as the new Orderable Item. In these cases, the kill logic will be the same as the set logic. The kill and set logic will simply pass in the Orderable Item to the routine that checks all IV Additives/IV Solutions/Dispense Drugs matched to the Orderable Item, does all the necessary updates (Inactivation date, Supply flag, Non-formulary, Base, Additive), and then sends the Master File Update to CPRS on that Orderable Item. This type of update occurs when the fields listed below are edited.

File 50:    DEA Special Hdlg

File 50:    Inactivation Date

File 50:    Application Packages’ Use

File 50:    Local Non-Formulary

File 50.7: Inactivation Date

File 52.6: Inactivation Date

File 52.6: Used in IV Fluid Order Entry

File 52.7: Inactivation Date

File 52.7: Used in IV Fluid Order Entry

The second situation occurs when pointers to the PHARMACY ORDERABLE ITEM file (#50.7) are changed. IV Additives, IV Solutions and the Dispense Drug always point to the same Orderable Item. That Orderable Item is, in turn, pointed to by the IV Additive or IV Solution. So, the fields that may be affected include the Orderable Item pointer in the DRUG file (#50) and the Generic Drug pointer in the IV ADDITIVES file (#52.6) and the IV SOLUTIONS file (#52.7).

File 50:     Orderable Item Pointer

File 52.6:  Generic Drug Pointer

File 52.7:  Generic Drug Pointer

The initial change is to make the Orderable Item pointers in the IV ADDITIVES file (#52.6) and the IV SOLUTIONS file (#52.7) uneditable. The software will now control those pointers.

#### Scenario 1: The Orderable Item Pointer Is Changed For A Dispense Drug

In Example 1, the Orderable Item pointer is changed for a Dispense Drug. In this case, any Orderable Item pointers must be updated for entries in the IV ADDITIVES file (#52.6) and the IV SOLUTIONS file (#52.7) that point to that Dispense Drug. After these pointers have been updated, the Orderable Item must be updated for the old Orderable Item with what will point to it after the matching. The Orderable Item must also be updated for the new Orderable Item after the matching. And these pharmacy Orderable Item updates must be sent to CPRS as part of the Master File Update. To accomplish this the following steps must be completed.

- Add a Cross-Reference on the Orderable Item pointer in the DRUG file (#50) that will hard set one Cross-Reference in the ORDERABLE ITEM file (#50.7) and two Cross-References in the DRUG file (#50) as follows.

Orderable Item:    ^PS(50.7,"A50",Orderable Item IEN, Dispense Drug IEN)=""

Drug file:             ^PSDRUG("A526", Dispense Drug IEN, Additive IEN,)=""

Drug file:             ^PSDRUG("A527", Dispense Drug IEN, Solution IEN,)=""

The Orderable Item Cross-Reference allows access to Dispense Drugs matched to an Orderable Item.  The two DRUG file (#50) Cross-References allow access to Solutions and Additives linked to Dispense Drugs. An "A50" Cross-Reference will also be added on the NAME field  (# .01) of the PHARMACY ORDERABLE ITEM file (#50.7) containing a "Quit" command for the set and kill logic for documentation purposes only.

When the Orderable Item pointer of a Dispense Drug changes, only one Cross-Reference is needed on that field to perform the following actions.

- Kill Logic: This command performs a hard kill of the "A50" Cross-Reference in the PHARMACY ORDERABLE ITEM file (#50.7) for that Dispense Drug using old value (X) and DA, where X equals the OI IEN and DA equals the Dispense Drug IEN. The two DRUG file (#50) Cross-References will not change.

After the hard kill is completed, a Master File Update is performed for the old Orderable Item. The logic for all Dispense Drugs/IV Additives/IV Solutions matched to the Orderable item is executed by looping the three Cross-References to find all entries in all three files matched to the Orderable Item. Also in the Kill logic, the Orderable Item pointer is set to null and the Orderable Item pointer Cross-Reference is killed for any IV Additives or IV Solutions matched to the Dispense Drug.

- Set Logic: Using the New Value (X), where X equals the OI IEN, the "A50" Cross-Reference is hard set in the PHARMACY ORDERABLE ITEM file (#50.7). The Master File Update is then performed for the new Orderable Item. The logic for all Dispense Drugs/IV Additives/IV Solutions matched to the Orderable Item is executed by looping on the three Cross-References to find all entries in all three files matched to the Orderable Item. The Orderable Item pointer and the Orderable Item pointer Cross-References are then hard set for all IV Additives and IV Solutions that have been matched to the Dispense Drug with new value (X).

Example 1:

Additives/Solution		Dispense Drugs:		Orderable Item:

IEN 3    points to =>           	IEN 100    points to =>		500

IEN 4    points to =>           	IEN 100    points to =>		500

IEN 5    points to =>           	IEN 100    points to =>		500

IEN 10  points to =>		IEN 200     points to =>                   	500

Cross-References are:	^PS(50.7,"A50",500,100)=""

^PS(50.7,"A50",500,200)=""

^PSDRUG("A526",100,3)=""

^PSDRUG("A526",100,4)=""

^PSDRUG("A526",100,5)=""

^PSDRUG("A527",200,10)=""

Orderable Item 500 is pointed to by Dispense Drugs 100 and 200, and by IV Additives 3, 4, and 5, and IV Solution 10.

(If the LOCAL NON-FORMULARY field (#51) in the DRUG file (#50) is edited, the software will obtain the OI pointer 500 and execute the OI logic by looping on 500 in the "A50" Cross-Reference of the PHARMACY ORDERABLE ITEM file (#50.7). As it references each entry, the OI logic is executed by looping on the “A526” and “A527” Cross-references on the DRUG file (#50) before going to the next Orderable Item pointer in the "A50" Cross-reference in the PHARMACY ORDERABLE ITEM file (#50.7). For Example 1 above, the software will find in the first  "A50" Cross-Reference for OI 500, Dispense Drug 100. The software will then loop through all the “A526” and “A527” Cross-References in the DRUG file (#50) to find the IV Additives 3, 4 and 5. In the second “A50” Cross-Reference for OI 500, Dispense Drug 200 is identified.  The software will again loop through any existing “A526” and “A527” Cross-references in the DRUG file (#50) to find IV Solution 10.

If the Orderable Item pointer for Dispense Drug 100 is edited from 500 to 600, the Cross-Reference in the DRUG file (#50) the following logic will be performed.

- Kill Logic

Kill the Cross-Reference ^PS(50.7,"A50",500,100) using DA and old value (X=500), where DA equals the  IEN of the Dispense Drug and X equals the IEN of the Orderable Item

The Cross-References would now be as follows.

^PS(50.7,"A50",500,200)=""

^PSDRUG("A526",100,3)=""

^PSDRUG("A526",100,4)=""

^PSDRUG("A526",100,5)=""

^PSDRUG("A527",200,10)=""

The ‘A50” and “A527” Cross-references now identify Orderable Item 500 to be pointed to by Dispense Drug 200 and IV Solution 10. The Orderable Item update for OI 500 is then performed for Dispense Drug 200 and IV solution 10.

While still in the Kill logic, the PHARMACY ORDERABLE ITEM field (#15) in the IV ADDITIVES file (#52.6) is set to null for IV Additives 3, 4, and 5. This action results in the deletion of Cross-References on the PHARMACY ORDERABLE ITEM field (#15) of the IV ADDITIVES file (#52.6).

#### Set Logic

The “A50” Cross-Reference in the PHARMACY ORDERABLE ITEM file (#50.7) for the new Orderable Item 600 is set as follows.

^PS(50.7,"A50",500,200)=""

^PS(50.7,"A50",600,100)=""

^PSDRUG("A526",100,3)=""

^PSDRUG("A526",100,4)=""

^PSDRUG("A526",100,5)=""

^PSDRUG("A527",200,10)=""

The Orderable Item logic is executed on the new OI 600 by looping on the "A50" Cross-Reference, to get the Dispense Drug pointer of 100. The software then loops through any existing “A526” and “A527” Cross-References to get IV Additives 3, 4 and 5.

The value of the PHARMACY ORDERABLE ITEM (#15) field in the IV ADDITIVES file (#52.6) for IV Additives 3, 4, and 5 is set to 600. Existing Cross-References are also set to reflect this change.

#### Scenario 2: The Dispense Drug Pointer Is Edited For An IV Additive Or  IV Solution

If the Dispense Drug is changed for an IV Additive or IV Solution, the Cross-References on the PHARMACY ORDERABLE ITEM field in the IV ADDITIVES file (#52.6) and IV SOLUTION file (#52.7) will perform the following set and kill logic.

- Kill Logic

First, the  "A526" or "A527" Cross-References in the DRUG file (#50) will be killed. Then, using DA, which is equal to the Orderable Item IEN, the software will get the old Orderable Item pointer value and perform the Orderable Item logic on the old Orderable Item. Subsequently, the value in the PHARMACY ORDERABLE ITEM field for the IV Additive and/or IV Solution will be set to null and the existing Cross-References on this field will be killed.

- Set Logic

First, the "A526" or "A527" Cross-References in the DRUG file (#50) will be set. Then Using X, which is equal to the Dispense Drug IEN, the software will identify the new Orderable Item in the DRUG file (#50) and perform the OI logic on that Orderable Item. The PHARMACY ORDERABLE ITEM field in the IV ADDITIVES file (#52.6) and IV SOLUTION file (#52.7) will be set to the new value and existing Cross-References will be also set.

Users can first check the new Dispense Drug, and if the Orderable Item does not change by rematching the Additive/Solution to the new Dispense Drug, they can choose the QUIT command.

Example 2:

#### IV Additives/IV Solution	Dispense Drugs		Orderable Item

IEN 3    points to =>           	IEN 100    points to =>		500

IEN 4    points to =>          	IEN 100    points to =>		500

IEN 5    points to =>         	IEN 100    points to =>		500

IEN 10  points to =>		IEN 200     points to =>                   	500

Cross-References 	^PS(50.7,"A50",500,100)=""

^PS(50.7,"A50",500,200)=""

^PSDRUG("A526",100,3)=""

^PSDRUG("A526",100,4)=""

^PSDRUG("A526",100,5)=""

^PSDRUG("A527",200,10)=""

For example, the USED IN IV FLUID ORDER ENTRY field (#17) in the IV ADDITIVES file (#52.6) for IV Additive 3 could be edited. The Orderable Item that the IV Additive points to in this case, is 500. Both the Kill and Set logic (same logic) for the OI 500 is updated by looping through the "A50" Cross-Reference in the PHARMACY ORDERABLE ITEM file (#50.7), finding each Dispense Drug IEN, and going through the "A526" and  "A527" Cross-References in the DRUG file (#50) for that Dispense Drug. This process is then repeated for the next Dispense drug identified in the “A50” Cross-Reference

If the DRUG file (#50) pointer for IV Additive 3 were changed from Dispense Drug 100 to Dispense Drug 900, the Cross-Reference on the Dispense Drug Pointer would be killed.

- Kill Logic

Using old value of X, which equals the Dispense Drug 100 and DA, which equals the IV ADDITIVE 3, the software would kill Cross-Reference  ^PSDRUG("A526",100,3) with the following Cross-References remaining.

^PS(50.7,"A50",500,100)=""

^PS(50.7,"A50",500,200)=""

^PSDRUG("A526",100,4)=""

^PSDRUG("A526",100,5)=""

^PSDRUG("A527",200,10)=""

Using DA, the software would get the old Orderable Item pointer of 500 and execute the Orderable Item logic for Dispense Drugs 100, IV Additives 4 and 5, Dispense Drug 200, and IV Solution 10.

The value for the PHARMACY ORDERABLE ITEM field (#15) in the IV ADDITIVES file (#52.6) would be set to null and Cross-References on this field would be deleted.

- Set Logic

Using new value X, where X equals the Dispense Drug 900, the software would set the new "A526" Cross Reference as ^PSDRUG("A526",900,3)="", The updated Cross-References are as follows

^PS(50.7,"A50",500,100)=""

^PS(50.7,"A50",500,200)=""

^PSDRUG("A526",100,4)=""

^PSDRUG("A526",100,5)=""

^PSDRUG("A526",900,3)=""

^PSDRUG("A527",200,10)=""

Using new value of X, where X equals the Dispense Drug 900, the software gets the Orderable Item pointer for Dispense Drug 900, in this example, Orderable Item 2000. The applicable Cross-References would be the following.

^PS(50.7,"A50",500,100)=""

^PS(50.7,"A50",500,200)=""

^PS(50.7,"A50",2000,900)=""

^PSDRUG("A526",100,4)=""

^PSDRUG("A526",100,5)=""

^PSDRUG("A526",900,3)=""

^PSDRUG("A527",200,10)=""

The software performs the OI update for Orderable Item 2000, with Dispense Drug 900 and IV Additive 3. The PHARAMCY ORDERABLE ITEM field (#15) value in the IV ADDITIVES file (#52.6) is set to 2000. The corresponding Cross-References on this field are also set.

Security Guide

(This page included for two-sided copying.)

## Security Management

The PDM package does not contain any VA FileMan security codes except for programmer security (@) on the data dictionaries for the PDM files. Security with respect to standard options in the module is implemented by carefully assigning options to users and by the use of security keys.

## Mail Groups

Patch PSS*1*147 creates a new mail group called PSS ORDER CHECKS. The mail group description below was retrieved from VA FileMan. The IRM Pharmacy support and Pharmacy ADPACs (and backups) should at a minimum be added to this mail group.

NAME: PSS ORDER CHECKS

TYPE: public

DESCRIPTION:   Members of this mail group will receive various notifications

that impact Enhanced Order Checks (drug-drug interactions, duplicate therapy

and dosing checks) introduced with PRE V. 0.5 utilizing a COTS database.

## Alerts

There are no alerts in the PDM package.

## Remote Systems

PDM does not transmit data to any remote system or facility.

## Archiving/Purging

There are no archiving and purging functions necessary with the PDM package.

## Contingency Planning

Sites utilizing the PDM package should develop a local contingency plan to be used in the event of product problems in a live environment. The facility contingency plan must identify the procedure for maintaining functionality provided by this package in the event of system outage. Field station Information Security Officers (ISOs) may obtain assistance from their Regional Information Security Officer (RISO).

## Interfacing

There are no specialized products embedded within or required by the PDM package.

## Electronic Signatures

No electronic signatures are utilized in the PDM package.

## Menus

The Pharmacy Data Management menu that is exported with the initial release of the PDM package is as follows.

*Primary/VA Generic Orderable Item Report

*VA Generic Orderable Item Report

*Create Pharmacy Orderable Items

*Manually Match Dispense Drugs

*Orderable Item Matching Status

CMOP Mark/Unmark (Single drug)

Locked with PSXCMOPMGR

Drug Enter/Edit

IMPORTANT: Once Pharmacy Data Management is installed, the Outpatient Drug Enter/Edit and the Inpatient Medications Dispense Drug Fields options will be disabled and the PDM Drug Enter/Edit option should be used.

Drug Interaction Management...

Enter/Edit Local Drug Interaction

Edit Drug Interaction Severity

Electrolyte File (IV)

Lookup into Dispense Drug File

Med. Route/Instructions Table Maintenance

Medication Instruction File Add/Edit

Orderable Item Management...

Edit Orderable Items

Dispense Drug/Orderable Item Maintenance

Additive/Solutions, Orderable Items

Orderable Item Report

*Primary Drug Edit

Pharmacy System Parameters Edit

Standard Schedule Edit

* These items are for pre-release only and will be deleted with the installation Outpatient Pharmacy V. 7.0 and Inpatient Medications V. 5.0.

The PDM menu up to and including the release of PSS*1*147 appears below. PSS*1*147 was the last patch to affect a change to the PDM menu.

CMOP Mark/Unmark (Single drug)

Dosages ...

Auto Create Dosages

Dosage Form File Enter/Edit

Enter/Edit Dosages

Most Common Dosages Report

Noun/Dosage Form Report

Review Dosages Report

Local Possible Dosages Report

Request Change to Dose Unit

Drug Enter/Edit

Drug Interaction Management ...

Enter/Edit Local Drug Interaction

Report of Locally Entered Interactions

Electrolyte File (IV)

Lookup into Dispense Drug File

Medication Instruction Management

Medication Instruction File Add/Edit

Medication Instruction File Report

Medication Routes Management

Medication Route File Enter/Edit

Medication Route Mapping Report

Medication Route Mapping History Report

Request Change to Standard Medication Route

Default Med Route for OI Report

Orderable Item Management ...

Edit Orderable Items

Dispense Drug/Orderable Item Maintenance

Orderable Item/Dosages Report

Patient Instructions Report

Orderable Item Report

Formulary Information Report

Drug Text Management

Drug Text Enter/Edit

Drug Text File Report

Pharmacy System Parameters Edit

Standard Schedule Management

Standard Schedule Edit

Administration Schedule File Report

Synonym Enter/Edit

Other Language Translation Setup

Controlled Substances/PKI Reports

DEA Spec Hdlg & CS Fed Sch Discrepancy

Controlled Substances Not Matched to NDF

CS (DRUGS) Inconsistent with DEA Spec Hdlg

CS (Ord. Item) Inconsistent with DEA Spec Hdlg

Send Entire Drug File to External Interface

**Enhanced Order Checks Setup Menu

Find Unmapped Local Medication Routes

Map Local Medication Route to Standard

Medication Route Mapping Report

Medication Route File Enter/Edit

Medication Route Mapping History Report

Request Change to Standard Medication Route

Find Unmapped Local Possible Dosages

Map Local Possible Dosages

Local Possible Dosages Report

Strength Mismatch Report

Enter/Edit Dosages

Request Change to Dose Unit

Mark PreMix Solutions

IV Solution Report

Administration Schedule File Report

Medication Instruction File Report

IV Additive/Solution Reports

IV Additive Report

IV Solution Report

Warning Builder

Warning Mapping

Locked: PSXCMOPMGR

Without the PSXCMOPMGR key, the CMOP Mark/Unmark (Single drug) option will not appear on your menu.

Other Language Translation Setup [PSS OTHER LANGUAGE SETUP] option is a stand-alone option that must be assigned to the person(s) responsible for maintaining it.

** This menu is for the Pharmacy Reengineering (PRE) Version 0.5 Pre-Release only and will be deleted with the installation of PRE V. 0.5.

## Security Keys

In order to mark or edit package specific fields in a DRUG file (#50) entry, the user must hold the corresponding package key. The keys are assigned for the individual packages. PDM does not export any of these keys.

Package	Keys

Outpatient Pharmacy	PSORPH

Inpatient Medications	PSJU MGR

Inpatient Medications	PSJI MGR

Automatic Replenishment/Ward Stock 	PSGWMGR

Drug Accountability/Inventory Interface	PSAMGR

Drug Accountability/Inventory Interface	PSA ORDERS

Controlled Substances 	PSDMGR

National Drug File 	PSNMGR

Consolidated Mail Outpatient Pharmacy 	PSXCMOPMGR

Patch PSS*1*147 exports the following four security keys, that will be used by the Pharmacy Enterprise Customization System (PECS) application. Only a few users who will be granted access to the PECS application will need one or more keys assigned based on their role. Assignment of these keys should be by request only. The security key descriptions were retrieved from VA FileMan.

NAME: PSS_CUSTOM_TABLES_ADMIN

DESCRIPTIVE NAME: ADMINISTRATOR

DESCRIPTION:   This key is used by the Pharmacy Enterprise Customization System (PECS) web application. Holders of this key will have the ability to perform configuration and administrative tasks for the application. They will also have querying capabilities.

NAME: PSS_CUSTOM_TABLES_APPROVER

DESCRIPTIVE NAME: APPROVER

DESCRIPTION:   This key is used by the Pharmacy Enterprise Customization System (PECS) web application.  Holders of this key will have the same privileges as those with the PSS CUSTOM TABLES REQUESTOR key. Additional capabilities will be to review, approve, delete or reject customization requests and to view and generate reports.

NAME: PSS_CUSTOM_TABLES_REL_MAN

DESCRIPTIVE NAME: RELEASE MANAGER

DESCRIPTION:   This key is used by the Pharmacy Enterprise Customization System (PECS) web application. Holders of this key will have the ability to create file updates for FDB database tables to be applied at local facilities. They will also have querying capabilities.

NAME: PSS_CUSTOM_TABLES_REQUESTOR

DESCRIPTIVE NAME: REQUESTOR

DESCRIPTION:   This key is used by the Pharmacy Enterprise Customization System (PECS) web application.  Holders of this key will be allowed to enter customization requests, display and view the status of their own requests. They will also have limited querying capabilities

## File Security

Information about all files, including these, can be obtained by using the VA FileMan to generate a list of file attributes.

#### PDM Files

|  | File Numbers | File Names | DD | RD | WR | DEL | LAYGO |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 DRUG @ |  |  |  |  |  |  |  |  |
| 50.4 DRUG ELECTROLYTES @ |  |  |  |  |  |  |  |  |
| 50.606 DOSAGE FORM @ @ @ @ |  |  |  |  |  |  |  |  |
| 50.7 PHARMACY ORDERABLE ITEM @ |  |  |  |  |  |  |  |  |
| 51 MEDICATION INSTRUCTION @ |  |  |  |  |  |  |  |  |
| 51.1 ADMINISTRATION SCHEDULE @ |  |  |  |  |  |  |  |  |
| 51.2 MEDICATION ROUTES @ |  |  |  |  |  |  |  |  |
| 51.23 STANDARD MEDICATION ROUTES @ Pp @ @ @ |  |  |  |  |  |  |  |  |
| 51.24 DOSE UNIT @ Pp @ @ @ |  |  |  |  |  |  |  |  |
| 51.5 ORDER UNIT |  |  |  |  |  |  |  |  |
| 51.7 DRUG TEXT @ |  |  |  |  |  |  |  |  |
| 52.6 IV ADDITIVES @ |  |  |  |  |  |  |  |  |
| 52.7 IV SOLUTIONS @ |  |  |  |  |  |  |  |  |
| 54 RX CONSULT |  |  |  |  |  |  |  |  |
| 55 PHARMACY PATIENT (Partial DD) @ P |  |  |  |  |  |  |  |  |
| 59.7 PHARMACY SYSTEM ^ ^ ^ ^ |  |  |  |  |  |  |  |  |

#### Non-PDM Files

| File Numbers | File Names | DD | RD | WR | DEL | LAYGO |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 200 NEW PERSON (Partial DD) # # # # # |  |  |  |  |  |  |  |
| 9009032.3 APSP INTERVENTION TYPE |  |  |  |  |  |  |  |
| 9009032.4 APSP INTERVENTION |  |  |  |  |  |  |  |
| 9009032.5 APSP INTERVENTION RECOMMENDATION |  |  |  |  |  |  |  |

Please refer to the "Sending Security Codes." section of the Kernel V. 8.0 Systems Manual for more information concerning installation of security codes.

## References

There are no regulations or directives related to the Pharmacy Data Management package. Additional manuals related to the Pharmacy Data Management package can be found at the VistA Documentation Library (VDL) on the Internet at: http://www.va.gov/vdl/.

(This page included for two-sided copying.)

# Glossary

Administration Schedule File	The ADMINISTRATION SCHEDULE file (#51.1) contains administration schedule names and standard dosage administration times. The name is a common abbreviation for an administration schedule (e.g., QID, Q4H, PRN). The administration time is entered in military time.

CPRS 	A VistA computer software package called Computerized Patient Record System. CPRS is an application in VistA that allows the user to enter all necessary orders for a patient in different packages from a single application.

Dispense Drug	The Dispense Drug is pulled from DRUG file (#50) and usually has the strength attached to it (e.g., Acetaminophen 325 mg). Usually, the name alone without a strength attached is the Pharmacy Orderable Item name.

Dosage Form File	The DOSAGE FORM file (#50.606) contains all dosage forms and associated data that are used by Pharmacy packages and CPRS. The dosage form is used in SIG construction, default values and in the determination of the type of each dosage created for each application.

Dose Unit File	The DOSE UNIT file (#51.24) was created to accomplish the mapping to First DataBank (FDB). All entries in this file have been mapped to an FDB Dose Unit. Although this file has not yet been standardized by Standards and Terminology Services (SRS), no local editing will be allowed. When populating the Dose Unit field for a Local Possible Dosage, selection will be from this new file.

Drug Electrolytes File	The DRUG ELECTROLYTES file (#50.4) contains the names of anions/cations, and their cations and concentration units.

Drug File	The DRUG file (#50) holds the information related to each drug that can be used to fill a prescription or medication order. It is pointed to from several other files and should be handled carefully, usually only by special individuals in the Pharmacy Service. Entries are not typically deleted, but rather made inactive by entering an inactive date.

Drug Interaction File	The DRUG INTERACTION file (#56) is used to store DRUG-DRUG interactions.  The file is sent out

with data pre-populated. The pre-populated data cannot be deleted, data can only be added, or the severity of the national data can be elevated locally.

Drug Text File	The DRUG TEXT file (#51.7) stores rapidly changing drug restrictions, guidelines, and protocols to help assure medications are being used according to defined specifications.

IV Additives File	The IV ADDITIVES file (#52.6) contains drugs that are used as Additives in the IV room. Data entered includes drug generic name, print name, drug information, synonym(s), dispensing units, cost per unit, days for IV order, usual IV schedule, administration times, electrolytes, and quick code information.

IV Solutions File	The IV SOLUTIONS file (#52.7) contains drugs that are used as primary solutions in the IV room. The solution must already exist in the Drug file (#50) to be selected. Data in this file includes: drug generic name, print name, status, drug information, synonym(s), volume, and electrolytes.

Local Possible Dosages	Local Possible Dosages are free text dosages that are associated with drugs that do not meet all of the criteria for Possible Dosages.

Medication Instruction File	The MEDICATION INSTRUCTION file (#51) is used by Unit Dose and Outpatient Pharmacy. It contains the medication instruction name, expansion and intended use.

Medication Routes File	The MEDICATION ROUTES file (#51.2) contains medication route names. The user can enter an abbreviation for each route to be used at their site. The abbreviation will most likely be the Latin abbreviation for the term.

Medication Routes/Abbreviations	The Medication RouteS file (#51.2) contains the medication routes and abbreviations, which are selected by each Department of Veterans Affairs Medical Centers (VAMC). The abbreviation cannot be longer than five characters to fit on labels and the Medical Administration Record (MAR). The user can add new routes and abbreviations as appropriate.

National Drug File	The National Drug File provides standardization of the local drug files in all VA medical facilities. Standardization includes the adoption of new drug nomenclature and drug classification and links the local drug file entries to data in the National Drug File. For drugs approved by the Food and Drug Administration (FDA), VA medical facilities have access to information concerning dosage form, strength and unit; package size and type; manufacturer’s trade name; and National Drug Code (NDC). The NDF software lays the foundation for sharing prescription information among medical facilities.

Non-Formulary Drugs	Drugs that are not available for use by all providers.

Orderable Item	An Orderable Item is pulled from the PHARMACY ORDERABLE ITEM file (#50.7) and usually has no strength attached to it (e.g., Acetaminophen). The name, with a strength attached, is the Dispense Drug name (e.g., Acetaminophen 325mg).

Orderable Item File	The ORDERABLE ITEM file (#101.43) is a CPRS file that provides the Orderable Items for selection within CPRS. Pharmacy Orderable Items are a subset of this file.

Pending Order	A pending order is one that has been entered by a provider through CPRS without Pharmacy finishing the order. Once Pharmacy has finished (and verified for Unit Dose only) the order, it will become active.

Pharmacy Orderable Item	The Pharmacy Orderable Item is used through CPRS to order Inpatient Medications and Outpatient Pharmacy prescriptions.

Pharmacy Orderable Item File	The PHARMACY ORDERABLE ITEM file (#50.7) contains the Order Entry name for items that can be ordered in the Inpatient Medications and Outpatient Pharmacy packages.

Possible Dosages	Dosages that have a numeric dosage and numeric Dispense Units Per Dose appropriate for administration.  For a drug to have possible dosages, it must be a single ingredient product that is matched to VA PRODUCT file (#50.68).  The VA PRODUCT file (#50.68) entry must have a numeric strength and the dosage form/unit combination must be such that a numeric strength combined with the unit can be an appropriate dosage selection.

Prompt	A point at which the system questions the user and waits for a response.

Standard Medication Route File	The STANDARD MEDICATION ROUTE file (#51.23) was created to map Local Medication Routes in VistA to an FDB Route in order to perform dosage checks in PRE V.0.5.  This file has been standardized by Standards and Terminology Service (STS) and is mapped to an FDB Route. It cannot be edited locally.

Standard Schedule	Standard medication administration schedules are stored in the Administration Schedule file (#51.1).

Units Per Dose	The Units Per Dose is the number of Units (tablets, capsules, etc.) to be dispensed as a dose for an order. Fractional numbers will be accepted.

VA Drug Class Code	A drug classification system used by VA that separates drugs into different categories based upon their characteristics. Some cost reports can be run for VA Drug Class Codes.

VA Product File	The VA PRODUCT file (#50.68) contains a list of available drug products.