CLINICAL PROCEDURES (CP) V1.0

FLOWSHEETS MODULE

RELEASE NOTES

MD*1.0*16
October 2010

Department of Veterans Affairs

Office of Information & Technology (OI&T)

Product Development (PD)

Revision History

| Description | Date | Author |
| --- | --- | --- |
| Patch MD*1.0*16 released. | October 2010 | Joe Niksich Brian Juergensmeyer |

Table of Contents

Release Notes for Patch MD*1.0*16	1

Overview	1

Patch MD*1.0*16 Features	1

Interface with Third Party Monitoring Devices	1

Terminology	1

GUI Flowsheets	2

ADT HL7 Message	2

Manual Data Entry	2

Data to CPRS	2

GUI Console	2

Background Tasks	2

Gateway Service	2

Implementation Considerations	3

Installation	3

Validated Patient Monitoring Devices	3

Additional Medical Monitoring Devices	4

List of Outstanding Anomalies	5

This page intentionally left blank for double-sided printing.

# Release Notes for Patch MD*1.0*16

Patch MD*1.0*16 releases a new Veterans Health Information Systems and Technology Architecture (VistA) system consisting of graphical user interface (GUI) components,  and is comprised ofthree GUI components: Clinical Procedures (CP) Gateway Service, CP Console, and CP Flowsheets.

- The CP Gateway Service application allows third party vendor devices to send observational data to a VistA CP system for display and reporting. VistA HL7 messaging is the receiver(generator) of the data passed from the devices to VistA.

- CP Console provides the tools to administer the Clinical Procedures (CP) package.  It will provide facilities to develop flowsheet views, totals, and flowsheets themselves.  It also provides the ability to administer legacy CP procedures and devices.

- CP Flowsheets provides an electronic format for the traditional paper flowsheet maintained during each inpatient stay.  It provides functionality such as data entry and validation, and HL7 error mitigation for CP.

## Overview

MD*1.0*16 provides an interface with which to collect and store patient observation information from ICU monitoring devices .

MD*1.0*16 is also capable of publishing validated information retrieved from third party devices in the Clinical Remiders Index.  This is accomplished using the CP Gateway Service and the new Clinical Observations (CliO) service infrastructure.

# Patch MD*1.0*16 Features

Patch MD*1.0*16 includes several features.

## Interface with Third Party Monitoring Devices

MD*1.0*16 provides an interface for the collection of patient observational data from  monitoring devices. The collected data is stored automatically in the appropriate VistA database(s).

## Terminology

MD*1.0*16 provides standardized terminology with VA Unique Identifiers (VUIDs).  This facilitates exporting CliO data to other VA systems that use standardized terminology.

## GUI Flowsheets

Clinical Flowsheets provides utilizes locally-customizable flowsheets  to view, enter, and edit patient data entered manually or received via the CliO infrastructure.  Ultimately, this application will provide the front-end for other clinical system, such as the Renal Dialysis, and Intake & Output (I&O) packages.

## ADT HL7 Message

Clinical Flowsheets introduces an Admission Discharge and Transfer (ADT) Health Level 7 (HL7) message feed. The ADT feed monitors the admitted, discharged, and transferred events from the Patient Information Management System (PIMS) and notifies interested patient medical monitoring devices.

## Manual Data Entry

CP Flowsheets allows you to manually enter data personally observed by clincial staff or collected by monitors that are not able to electronically send that information.

## Data to CPRS

MD*1.0*16 is capable of publishing data to CPRS in two ways.   Vitals observations entered into the CliO service architecture is direcdtly viewable in Vitals/Vitals Lite in CPRS.  All other observational data entered may be published in a TIU note, which is then viewable in CPRS.

## GUI Console

CP Console is a user-friendly GUI with which you configure system parameters and patient medical monitoring devices, schedule background tasks and shifts, and design flowsheet templates to meet site-specific requirements.

## Background Tasks

CP Console provides specific background tasks to purge and clean up the system with regard to processed flowsheet data and legacy CP studies.

## Gateway Service

The new CP Gateway Service is a service that allows patient monitoring devices to send observational data to VistA.

# Implementation Considerations

## Installation

To install patch MD*1.0*16, follow the Clinical Procedures (CP) V1.0 Flowsheets Module Installation Guide carefully. Although part of CP, Clinical Flowsheets is independent of CP User, CP Manager, CP Gateway, and CP Hemodialysis.

- Vitals patches GMRV*5.0*22, GMRV*5.0*23, and MD*1.0*21 must be installed prior to the installation of the MD*1.0*16 software.

- MD*1.0*16 includes pre-install routine MDPRE16 and post-install routine MDPOST16.  Verify that these routines are complete before attempting to use Clinical Flowsheets.

## Validated Patient Monitoring Devices

A supported device is one that has its data elements mapped to the standard terminology of the CliO database.  The following devices are currently supported:

- Aware Gateway (GE Healthcare)

- Intesys Clinical Suite (Spacelabs)

- OPV-1500k (Nihon Koden)

- PC1 (Spacelabs)

- PC2 (Spacelabs)

- Ultraview 1050 (Spacelabs)

- Ultraview 1600 (Spacelabs)

- Ultraview 1700 (Spacelabs)

- Ultraview SL2200 (Spacelabs)

- Ultraview SL2400 (Spacelabs)

- Ultraview SL2600 (Spacelabs)

- Ultraview SL2700 (Spacelabs)

- Ultraview SL2800 (Spacelabs)

- Ultraview SL3800 (Spacelabs)

## Additional Medical Monitoring Devices

ICU devices will be added to the list of validated instruments as the devices are certified through the Hines Field Office.

For sites that want to implement ICU devices, but do not have Clinical Procedures implemented,  refer to the Clinical Procedures (CP) V1.0 Flowsheets Module Implementation Guide. You do not need to implement the legacy Clinical Procedures.

For sites that use legacy Clinical Procedures and want to implement the Clinical Flowsheets patch, continue using the legacy CP modules and use CP Console to configure the ICU devices.

Note:  The CP Manager application is no longer supported after the installation of MD*1.0*16.  Use CP Console to perform the functions previously provided by CP Manager.

## List of Outstanding Anomalies

A Request for Concurrence to Release Software with Known Anomalies was submitted containing the following anomalies:

| id | Headline/Description | Screenshot Location | Severity | Priority | Application | Workaround/ Comments | Resolution |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Vital192 | Deleting and Renaming Flowsheets, Totals, Background Tasks, & Parameters - I can delete and rename (buttons enabled) a flowsheet and a flowsheet total, but I can only rename (button enabled) a flowsheet view. Should I be able to delete (enable button) a view as well? I cannot delete or rename (buttons disabled) the three background tasks and one of the parameters, but I can delete (button enabled) the ADT and gateway parameters. I would expect that I would not be able to delete ADT and gateway as well. |  | 3 - Average | 3 - Normal Queue | CP Console | Working as Designed per requirement not fixed; cannot delete flowsheet view; cannot delete or rename background tasks; cannot rename all Parameters; cannot delete or rename CP Parameters. | Version 2. |
| Vital213 | Many Opportunities to Add or Alter the Display Name for Various Items - When creating a new Flowsheet, you have to give it a display name. In a new session of CP Flowsheets, click Add Page and the Flowsheet Page Management pops up the Display Name from CP Console and displays as Page Name. Then I must add another Display Name and Comment in order to activate it. After activating it both the Display Name from CP Console and the one entered on the Flowsheet Page Management pop-up displays. If I enter the same Display Name in both places, both will display on the view/page. In both applications, there are many opportunities to add or alter the Display Name for the various items. It is confusing to have so many places to enter a Display Name whether adding for the first time or changing it. | N/A | 4 - Minor | 3 - Normal Queue | CP Console | To avoid user confusion, will be updated for version 2 but this is working as designed. | Will be addressed in Version 2.0 scope and prioritized by VHA stakeholders |
| Version 2 #1 | Note to Clinicians that the following data links are not present in Clinical Flowsheets version 1, as they were outside the scope of the project: - Vitals entered using CPRS Vitals Lite, will not appear in Clinical Flowsheets - Data entered in Clinical Flowsheets will not be forwarded to HDR - I & O data entered using Clinical Flowsheets will not be visible in Vitals Lite |  | N/A | N/A | Clinical Flowsheet GUI | Scope issues per Version 1 requirements and scope | Will be addressed in Version 2.0 scope and prioritized by VHA stakeholders |